# Forward Keying Description.

## Preparing.

Load Background Image to channel B and clear Alpha channel or fill Alpha channel by corresponding image. Select Live video on channel A or load Actor image to channel A. Expand Forward Control Panel by button .

Keying process include following steps:

1) Backing (green or blue) colors selection.

2) Actor colors selection.

3) Key-table loading.

4) Keying parameters setting.

5) Computer background changing.

6) Special case of alpha channel usage.

7) Advanced turning of key level.

## Backing (green or blue) colors selection.

Use button  to capture live image. Image for keying consists from two parts: backing (green area) and actor (women). To select backing colors you need press button  and select backing area on captured image by left mouse button:

Attention: Do not select area near actor edge or on transparent objects. You need select backing area only. Easiest way is capture image without any actor and select whole image.

## Actor colors selection.

To select actor colors you need press button  and select actor area on captured image by left mouse button:

Attention: Do not select area near backing edge or on transparent objects. You need select actor inside area only. If your actor will turn around, walk or you need key some actors then you need sequentially capture images with each actor (or actor position) and select new colors on each captured image. You need specify all different colors on all actors.

## Key-table loading.

To calculate key-table and load it into FORWARD device press button . After changing backing or actor colors or any key parameters you need press this button again.

To view keying result on FORWARD device press button  in Mixer group. To view keying result on Forward Control Panel press button  and release buttons  and  above captured image.

## Keying parameters setting.

Keying parameters are placed in "Color key" group:

Main parameter is color space model: "Bits 6-6-6", "Bits 4-7-7" or "Bits 2-8-8". Model 6-6-6 is more suitable for all backing colors.

You can select key-mask filtering type: "Filter 0" (no filtering), "Filter 1" (soft filtering) or "Filter 2" (hard filtering).

All colors are present on colored rectangle at left edge of group. Light blue area presents selected backing colors. Light red area presents selected actor colors. Dark blue area around light blue area is backing noise area, witch specified by slider . Dark red area around light red area is actor noise area, witch specified by slider . After changing sliders you need press button  to recalculate key-table.

You can clear actor colors you need press button  and clear colors by right mouse button. To clear backing colors you need press button  and clear colors by right mouse button. Right vertical blue slider with numbers from 0 to 255 represents working brightness interval.

Buttons  and  allow you select backing replacing mode. If both buttons are not pressed then used standard mode: image from channel A (live video) is mixed with image from channel B (computer decoration) by key-mask. In this mode actor edge has areola of backing color. If button  pressed then used enhanced mode when backing color from channel A replaced by image from channel with proportion of key-mask. In this mode actor has mach small colored areola then at first mode, but backing noise from channel A is added to resulting image. If pressed button  then backing noise is not added to resulting image.

Slider  allows you automatically non-select actor colors near backing colors. It specifies distance around backing colors (light blue area) where actor colors will never selected. After changing this slider you need press button  to recalculate key-table.

## Computer background changing.

FORWARD device can place actor into two-plane decoration. It is ordinal image with alpha channel witch specifies foreground area:

As result, FORWARD places actor between background and foreground planes:

Alpha channel has 8-bit per pixel color resolution. Alpha value less then 128 specifies background area. Respectively, alpha value more then 127 specifies foreground area in two-plane computer decoration.

FORWARD device can make color-keying together with image loading or AVI playback. You can render animated decoration and playback it by Forward Control Panel or by Forward AV Studio application. Remember that Forward AVI Player and Forward AV Studio will clear alpha channel before playback. Also, they will change Forward mixer state at starting and stopping playback. To avoid this we recommend fixing alpha channel and mixer by pressing on button  for Mixer and Alpha groups (button  will be changed to ).

## Special case of alpha channel usage.

In some case image from channel A (live video) has invalid areas: invalid backing or actor areas, witch is colored as background color. FORWARD has special tools for such cases.

### Invalid (small) backing.

If you have bad area of keying image without backing or with bad lighting but you know that actor will never appear on this area then you can fill alpha channel for this area by value 255. You need select alpha viewing (press button  or  above captured image), select mode "Suppress key for Bank B" (press button ) and draw invalid area by left mouse button. FORWARD will show channel B (computer decoration) inside drawing area.

### Bad colored actor (with colors near backing color).

If you have some area on actor with colors equal or near background colors and you know that this area will not moved then you can specify that this area always will be showed as channel A only. You need select mode "Suppress key by alpha" (press button ), then select alpha viewing (press button  or  above captured image), select mode "Suppress key for Bank A" (press button ) and draw invalid area by left mouse button. FORWARD will show channel A (live video) inside drawing area.

It may be useful when you have flag (or other colored picture) witch is painted on table or tribune. This object will not moved during shooting.

## Advanced turning of key level.

You can turn key level more carefully by special tool - key correction table. It may be necessary to remove some colored edges of actor ("blue-spill correction"). Press button , and following tool will appear:

Blue triangle at bottom indicates original key level when you move mouse across source image in main control panel window. Zero key level at left side corresponds to actor colors. Maximum key level at right side corresponds to backing colors. Click left mouse button to create new point in correction table. To move key point press left mouse button and shift it. To delete key point click right mouse button on this key point. If you will shift red line above diagonal then corresponding pixels at result image will have less backing colors. And if you will shift red line below diagonal then corresponding pixels will have more backing colors.

Don't forget to press button  after key correction table changing.