This file contains installation and troubleshooting information.

# INTRODUCTION

Forward is PCI Board. It require only one IRQ for audio and video input and output. Although the PCI standard is supposed to be a "plug-and-play" design, your computer may require some setup of the BIOS settings after

you've installed the Forward board. For Example, with some old BIOS you may have to use the Setup function to assign an IRQ (interrupt) number to the slot in which you've installed the Forward board. You'll first have to find an unused IRQ number, which you can do by running the "Start/Setting/Control Panel/System" on Windows 9x or "NT Diagnostic" on Windows NT.

The Setup program is usually entered by pressing the "Delete" key, while your computer is starting up (before DOS or Windows is booted). The specifics of the Setup program vary with every version of BIOS software, so you may have to consult the documentation provided with your computer's mainboard.

# SYSTEM REQUIREMENTS

- Forward Board works on computers with PCI Frequency up to 33MHz. Some BIOSes can set non-standard frequencies. That leads to PCI Frequency more than 33MHz. With such non-standard computer configurations Forward will not work.

- Interrupts on PCI must be activated by "Level". It is standard for PCI. See "BIOS Setup/PCI and PnP Configuration" or similar Page in PC hardware BIOS Configuration.

- Check in "BIOS Setup/PCI and PnP Configuration" that all Interrupts numbers owned by non PnP ISA Devices are set accordingly. Usually it concern ISA Sound and Network Cards.

- The PCI slot you've chosen for the Forward board is capable of "Bus Mastering". According to the PCI spec, all slots are supposed to be Bus Master capable, but some mainboards have one or two slots that are not. You'll have to check your mainboard documentation to find out.

- Do not try to get off doughtier board from Forward main board. You may not correctly restore it. Hence it will not function.

- Turn off power for Computer and other devices connected to Forward while you change video/audio interconnection.

- Windows 95/98 or Windows NT 4.0.SP3.

- At least 32Mb of RAM.

- Super VGA display (At least 800 x 600 x 64k colors)

- At least 20Mb of available system hard drive space for Forward Software.

- At least 1Gb of available system hard drive space for audio and video recording.

# RECOMENDED CONFIGURATIONS

- 64Mb of RAM.

- Super VGA display (At least 1024 x 768 x 64k colors)

- New IDE or SCSI disk with capacity at least 4Gb. Cache size must be at least 512Kb on Disk. See Disk Documentation.

- MainBoard on Intel VX,HX,TX,LX,BX Chipsets.

- CPU - Pentium 200MMX or better, Pentium II 266 or better. If you have Pentium II 350 or better and BX Chipset then set CPU BUS Frequency to 100MHz. Forward will work with Pentium II Celeron but will not achieve full performance. Celeron create too much traffic on Memory BUS!

- Forward support IRQ Sharing (one IRQ for more then one PCI board). But in a number of cases other Software does not do it correctly. So It is recommended to have own IRQ Number for each PCI device.

# INSTALLATION

## Driver Installation for Windows 95

- Windows 95 will automatically detect the Forward as a PCI Multimedia Video Device.

- When the "New Hardware Found" dialog box prompts for a driver, select "Driver from disk provided by hardware manufacturer" and insert the "Forward Software" CD into the CD drive.

- Select the correct path to this CD and click "OK".

- The "Select Device" dialog box should show the "Forward Video Capture and Playback Board" highlighted.

- Click "Next". The driver files will then be copied.

- If prompted to restart the computer, click "Yes".

## Driver Installation for Windows 98

- Windows 98 should automatically detect the Forward as a PCI Multimedia Video Device.

- When the "Add New Hardware" dialog box prompts for a driver, select "Display a list of all the drivers in a specific location, so you can select the driver you want." and click on "Next".

- Select the "Sound, Video and Game Controllers" and click on "Next".

- Click on  "Have Disk".

- Insert the "Forward Software" CD into the CD drive.

- Select the correct path to this CD and click "OK".

- The "Select Device" dialog box should show the "Forward Video Capture and Playback Board" highlighted.

- Click "OK". The driver files will then be copied.

- If prompted to restart the computer, click "Yes".

## Driver Installation for Windows NT 4.0

- Drivers for Windows NT 4.0 installed by Application Setup. See below "Applications installation on Windows 95/98/NT".

## Driver Update.

- You can update drivers simply installing new "Forward Software" over previous Installation. For Windows NT it is existing method only. See below "Applications installation on Windows 95/98/NT". This method is recommended because Applications require Drivers with specific version.

- For Windows 95/98 you can use standard driver update procedure.

## Driver Update for Windows 95/98

- In the Control Panel, click on "System".

- Select the "Device Manager" tab.

- Double click "Sound, Video and game controllers".

- 4.Select the "Forward Video Capture and Playback Board".

- Click on "Properties".

- Select the "Driver" tab.

- Click "Change Driver".

- Click on "Have Disk".

- Insert the "Forward Software" CD into the CD drive.

- Select the correct path to this CD and click "OK".

- The "Select Device" dialog box should show the "Forward Video Capture and Playback Board" highlighted.

- Click "OK". The driver files will then be copied.

- When prompted, click "Yes" to restart the computer.

## Applications installation on Windows 95/98/NT

- Insert the "Forward Software" CD into the CD drive.

- Run "Setup" from this CD.

- Setup starts and shows "Welcome" Dialog.  Click on "Next".

- Setup shows "Choose Destination Location" Dialog. If you do not like default Destination Folder then click on "Browse", select Destination Folder and click "OK". If Folder, you set, is not exist, Setup will ask you if it can create folder. Click "Yes".Click "Next".

- Setup shows "Setup Type" Dialog. Select Setup Type and click "Next". Setup types are:

- Compact		Only Applications and Windows NT Drivers will be installed.

- Custom			You can select what to install. See below.

- Forward Video Codecs	Only Video for Windows Codecs for UYVY,

Forward JPEG and Forward JPEG+Alpha formats will be installed. It is needed when you want to work with Forward AVIs on computers without Forward Board installed.

- Typical			Setup Type is suitable for most of users.

Applications, Windows NT Drivers and small number of samples will be installed.

- If you select "Custom" Setup, then you will see "Select Components" Dialog. Check components you want be installed and click "Next"

- Program Files		Application executables and help files. Recommended.

- Example Files		Examples for Applications. Recommended.

- Additional Examples	Additional Examples for Applications.

- Tutorial AVIs		Tutorial Films about Applications usage.

- Documents		Additional Documentation.

- NT Drivers		Setup shows it if you run Setup under Windows NT.

Recommended if you Installing Software on computer containing Forward Board.

- Setup shows "Select Program Folder" Dialog. Default is "Forward Software". If you want Select from existing or type other Program Folder Name. Click "Next".

- Setup shows "Start Coping Files" Dialog. You can see current setting and information about size of different components been installed and total size. Setup always tries to update Drivers. So you will see "Forward Drivers Update" always. If you click "Next", Setup will install Forward Software you select.

- When Installation is complete, restart Windows.

# PROBLEMS TROUBLESHOOTING

1. No video displayed by "Forward Video Capture" application or Video has no colors or distorted:

Be sure your video source (VTR, video camera, etc.) is properly connected to your Forward Board before recording. Then in "Forward In/Out Settings" Program verify that you have selected correct input port (VHS1, VHS2, VHS3   ... S-VHS 13, ..., YUV 156,...). Numbers and Colors on panel correspond to numbers and colors of input cables. Then check that input Video Standard is correct.

2. No audio recorded:

Be sure your audio source is properly connected to your computer before recording. Check that you enable audio recording in "Capture Video/Settings". Check that you select correct device as default in "Start/Settings/Control Panel/Multimedia/Sound". Check that you enable correct input in Recording Settings of "Volume Control" Windows Application.

3. The VTR is not responding:

Make sure that your serial control cable is properly connected to your computer before you select VTR mode from the player.  Make sure that your deck is in remote mode and not local. Confirm that your COM port settings are correctly enabled for your system. Make sure that the RS-232/422 adapter is firmly connected and receiving power.