How to Use API for Forward Operation.

General Structure.

At present IFrwPlayServer is a hi-level server. It is based on the low-level server frwdhsrv that operates directly with the hardware.

The main object of the low-level server is the IFrwBoard object, it realizes the following interfaces: IFrwBoard, IFrwInput, IFrwOutput, IFrwInterconnect, IFrwMixer, IFrwKeyer, IFrwPlay, IFrwGraber, IFrwRWData and IFrwGPIIO. These are the basic operational interfaces for parameters setup, data viewing, etc. More over, the low-level server contains the IFrwDriver object for creating the IFrwBoard object sample for the specific Forward board (it's implied that there may be more than one Forward board in a computer), and the IFrwFifo object. The IFrwFifo interface is intended for various video data playback (JPEG, alpha, uncompressed image, controls).

The hi-level server contains several objects. The main object IFrwPlayServer repeats most of the functions of the low-level server IFrwBoard object, it allows to create objects for video data playback and simply show the image. The hi-level server also includes a number of objects for video data manipulating: IFrwPlayYCrCbImage (uncompressed image), IFrwPlayJPEGImage (image compressed by Forward JPEG codec), IFrwPlayMovie (movie), IFrwPlaySuperClip, IFrwPlayTimeline and IFrwPlaySuperClipPlayer. The first three objects operate with the file on the disk (they have data download functions from the file on the disk). The last three – for object sequence playback of the first three types. IFrwPlaySuperClip allows to assemble together several fragments from different objects of the same type (for example, several AVI-files). IFrwPlayTimeline enables to playback the sequence of the IFrwPlaySuperClip objects (of different types) in real time. The object IFrwPlaySuperClipPlayer minimizes the developer's work at playback of one object IFrwPlaySuperClip.

API was created considering that the overwhelming majority of typical tasks can be realized using only hi-level server. The low-level server user has to understand the Forward structure and the operational methods. Hereinafter, the whole text assumes the use of the hi-level server. At first, you should create IFrwPlayServer object. In Visual Basic, it can be done in the following way:

Global ForwardServer As Object

Set ForwardServer = CreateObject("FrwPlayServer.FrwPlayServer")

In MSVC:

CComPtr<IFrwPlayServer> Server;

hres = Server.CoCreateInstance(CLSID_FrwPlayServer,NULL,CLSCTX_INPROC_SERVER);

It is more suitable to make global object and delete it only at the end of the program operations. It is required to specify the index of the board for operation with it. Later on NNN will be used instead of the index (0 by default).

Switching of the Operation Modes of Forward Board

(analog Forward Control Panel)

In order to switch the modes, it is required to create the hi-level server IFrwPlayServer and call the related functions with corresponding parameters. All these functions may be called from Visual Basic.

| To switch the mixer to | In Basic type in the lines |
| --- | --- |
| Bank A | ForwardServer.MixerValue(NNN, vdc_mixer_blender) = mixer_blender_0_ |
| Bank B | ForwardServer.MixerValue(NNN, vdc_mixer_blender) = mixer_blender_1_ |
| Mixing via alpha | ForwardServer.MixerValue(NNN, vdc_mixer_blender) = mixer_blender_direct_ ForwardServer.MixerValue(NNN, vdc_mixer_alpha) = mixer_alpha_ |
| Keying | ForwardServer.MixerValue(NNN,vdc_mixer_blender) = mixer_blender_direct_ ForwardServer.MixerValue(NNN,vdc_mixer_alpha) = mixer_keyer_ |

| To show live video | In Basic type in the lines |
| --- | --- |
| In bank A | ForwardServer.InterconnectValue(NNN, vdc_source_a) = source_for_a_video_ ForwardServer.MixerValue(NNN, vdc_mixer_a) = mixer_a_a_ ForwardServer.MixerValue(NNN, vdc_mixer_blender) = mixer_blender_0_ |
| In bank B | ForwardServer.InterconnectValue(NNN, vdc_source_b) = source_for_b_video_ ForwardServer.MixerValue(NNN, vdc_mixer_b) = mixer_b_b_ ForwardServer.MixerValue(NNN, vdc_mixer_blender) = mixer_blender_1_ |

Mixer_alpha_ and other constants are specified in the "frwdconsts.h" file. For more detailed study, it is recommended to use the example of the test application FRWDTEST. It’s possible to toggle the modes, and study what will happen in every individual case.

How to Disable the Mixer Switching

Most of the applications (and the viewing functions IFrwPlayServer and IFrwPlaySuperclipPlayer) switch the Forward board output so, that the result of their work can be viewed. For instance, the image display function (IFrwPlayServer.ShowYCrCbImage) at image viewing with alpha channel switches the mixer to mixing Bank A and Bank B by Alpha. In order to disable it, it is required to capture the mixer resource for duration of ShowYCrCbImage function realization. In Visual Basic, the following lines should be typed in:

ForwardServer.LockResource NNN, rg_mixer_bit_

ForwardServer.ShowYCrCbImage NNN, ...

ForwardServer.FreeResource NNN, rg_mixer_bit_

Notice that the hi-level server captures only one resource for a time. The low-level server can operate with the resources mask.

How to Playback the AVI-files Sequence

At first you should create the object IFrwPlaySuperClip:

Dim SuperClip As Object

Set SuperClip = Forward.SuperClip(FrwPlayMovie, FrwPlayShowAlphaB)

Then add all AVI-files into this object:

Dim Movie As Object, Time As Double

Set Movie = CreateObject("FrwPlayServer.FrwPlayMovie")

Movie.OpenFile "NextAviFile.avi"

Movie.GetMovieInfo 0, 0, 0, 0, Time, 0, 0, 0

SuperClip.AddMovieFragment Movie, 0, Time

Notice that at the example the object is asked it's duration (in seconds). You can add only a fragment of the whole movie into SuperClip. And the OpenFile function of the Movie object can be called only once. If you need to embed several fragments, then it would be easier to specify (create) the Movie variable within the cycle body.

Then create the IFrwPlaySuperClipPlayer object, and playback with it the gathered AVI-files sequence

Dim Player As Object

Set Player = Forward.SuperClipPlayer(NNN, SuperClip, 1, FrwPlayMovie, FrwPlayShowAlphaB, 0, 0)

Player.state = FrwPlaybackPrepared

Player.state = FrwPlaybackPlaying

Player.WaitState FrwPlaybackStopped

If you want to disable the mixer switching at playback start, you should enclose the mixing command of the player state into the playback mode with the capture controls of player resource.

ForwardServer.LockResource NNN, rg_mixer_bit_

Player.state = FrwPlaybackPlaying

ForwardServer.FreeResource NNN, rg_mixer_bit_

The Player.WaitState command would stop the program operation until the player moves to the specified state (in this example, until playback completion).

The Installation and the Use.

Forward Software Installer version 1.25 (and later) automatically installs Forward Server software. You need to install Forward Software before usage of Forward Server API.

If you use MS Visual C++ then you need include to your project files from INCLUDE directory.

If you use Borland Delphi or Borland Builder then you need use menu command "Project | Import Type Library".

If you use Visual Basic then you can use objects browser of Visual Basic to facilitate the operation in it. In order to fulfil it select the command "Service | References" and enable the references to "FrwPlayServer" (and "frwdhsrv") libraries. Then run the object browser (hotkey "F2"); there in the first list select the "FRWPLAYSERVERLib" library, choosing the class and concrete method – you may read brief information: what this method performs and what parameters it requires.