FDExtRender SDK

FDExtRender SDK – the brief description	2

FDExtRender SDK – interfaces	3

IFDExtMgr	3

IFDVideoOut	4

IFDSoundOut	5

Appendix	7

Example: select video player 	7

# FDExtRender SDK – the brief description

The package of software developer FDExtRender SDK is intended for simple playback video and audio data with minimal delay.

To use SDK import COM library FDExtSrvd into your project. For example, include follow code into your file "stdafx.h":

// FDExtSrvd

#import "libid:C03DE31D-62BA-4114-AB08-06F493FB98FF" named_guids raw_interfaces_only

Don’t forget initialize COM library at the beginning of any thread by call CoInitialize(NULL) or CoInitializeEx(NULL, COINIT_MULTITHREADED) and then uninitialized it on the finish by CoUninitialize().

Important Note: To enable playback on any FDExt board output by FDExtRender SDK you have to disable title mixer on the output! Start FDConfig2 and turn off check box "title mixer…" for corresponding output, then click OK to restart FDExtSrvs service (or restart PC).

There are two objects for playback video and audio data and one object for player selection.

Object FDExtMgr has interface IFDExtMgr witch allow to enumerate all valid players and access to the player.

Object FDVideoOut with interface IFDVideoOut allow playback video and audio data.

Object FDSoundOut with interface IFDSoundOut allow playback audio data.

# FDExtRender SDK – interfaces

## IFDExtMgr

Object: CLSID_FDExtMgr

Interface IFDExtMgr allow open video or audio player.

HRESULT ConnectRemoteHost([in] BSTR HostName) – connect to remote PC with specified name (or IP address) "HostName".

HRESULT get_FDObjectsCount([out, retval] long* pVal) – returns number of all objects in the system.

HRESULT get_FDObjectInfoByIndex([in] long Index, [out, retval] CFDObjectInfo* pFDObjectInfo) – return information about object with specified index. See the note 1.

HRESULT TranslateFDObjectInfo([in] unsigned long LocaleID, [in, out] CFDObjectInfo* pFDObjectInfo) – translate friendly names into specified structure into specified language (support Russian only).

HRESULT GetObjectFullName([in] CFDObjectInfo* pFDObjectInfo, [out] BSTR* pName) – return full name of the object.

HRESULT get_FDObjectByIndex([in] long Index, [in] CFDObjType FDObjType, [out, retval] IUnknown** ppUnk) – open COM-object with specified index and specified type and return it as IUnknown.

HRESULT get_FDObjectByName([in] BSTR Name, [in] CFDObjType FDObjType, [out, retval] IUnknown** ppUnk) – open COM-object with specified full name and specified type and return it as IUnknown

HRESULT get_NextValidFDObjectInfo([in, out] long* pNextIndex, [in] CFDObjType FDObjType, [out, retval] CFDObjectInfo* pFDObjectInfo) – return information about valid object with specified type. Method start searching from specified index *pNextIndex and return into pNextIndex incremented index of valid object. It there is no more valid objects with specified type then method return S_FALSE (not S_OK). This method used as another way to enumerate objects with specified type (see the note 2).

Other methods of the interface are not useful for FDExtRender SDK.

Notes:

- Structure CFDObjectInfo has follow fields:

long m_Valid 	should be 1 for valid object.

FDObjType m_FDObjType 	type of the object. Should be FDObjType_VideoOut (3) for video player or FDObjType_SoundOut (17) for audio player.

unsigned short m_FDObjRegName[128] 	unique name of the object (without board name).

unsigned short m_FDObjName[128] 	object name.

unsigned short m_FDObjFriendlyName[128] 	friendly name of the object.

long m_BoardIndex 	index of the FDExt board.

long m_IndexInBoard 	index of the object into the FDExt board.

long m_ObjTypeChannel 	not used.

long m_SoundChannels 	not used.

unsigned short m_BoardName[128] 	FDExt board name.

unsigned short m_BoardType[128] 	name of FDExt board type.

- To enumerate objects with known type you can use method get_NextValidFDObjectInfo. Call it while it return S_OK. Start from zero index. You should decrease returned index:

CFDObjectInfo info;

long nIndex = 0;

while( S_OK == pIMgr->get_NextValidFDObjectInfo(&nIndex,

FDObjType_VideoOut,

&info) )

{

// add one more valid video player for next usage

MyAddPlayer(nIndex-1, &info);

}

}

## IFDVideoOut

Interface IFDVideoOut allow playback video and audio on FDExt board output.

HRESULT FDGetVideoParams([out] CBoardVideoParams* pVideoParams) – return format of video data (see the note 1). Programs have to add video data with specified with, height and field order only.

HRESULT FDGetSoundParams([out] CBoardSoundParams* pSoundParams) – return format of audio data (see the note 2). Programs have to add audio data with specified ratio only.

HRESULT FDGetPreferredFormats([out] CFDCaptureVideoFormat* pVideoDataFormat, [out] CFDCaptureSoundFormat* pDataSoundFormat) – return preferred pixel format and audio sample format. It is native format of FDExt board.

HRESULT FDAllocateVideoFrame([in] uint64 FrameNumber, [in] uint64 WSSFlags) – allocate memory for next frame with video and audio data. Frame will be played at specified time with specified aspect ratio. See the note 3.

HRESULT FDWriteVideoFrameData([in] unsigned char* Buffer, [in] unsigned long cb, [in] long stride, [in] CFDCaptureVideoFormat DataFormat) – copy full frame video data from specified buffer into output fifo. Parameter "Buffer" specifies pointer of frame video data. Parameter "cb" specifies size (in bytes) of frame video data. Parameter "stride" specifies offset (in bytes) from one line to another into frame video data. Parameter "DataFormat" specifies pixel format.

HRESULT FDWriteVideoFieldData([in] unsigned char* Buffer, [in] unsigned long cb, [in] long stride, [in] CFDCaptureVideoFormat DataFormat, [in] char UpperField) – copy video data of one field from specified buffer into output fifo. Last parameter "UpperField" should be 1 for upper field and 0 for lower field.

HRESULT FDWriteSoundData([in] unsigned char* Buffer, [in] unsigned long cb, [in] CFDCaptureSoundFormat DataFormat, [in] unsigned long ChannelMask) – copy audio data for specified audio channels into output fifo. Duration of the audio data should correspond video duration with one sample accuracy (+/- one sample). Parameter "Buffer" specifies pointer of audio data. Parameter "cb" specifies size (in bytes) of audio data. Parameter "DataFormat" specifies sample format. Parameter "ChannelMask" specifies mask for output audio data. For example first language output should has mask 0x03, second language output should has mask 0x0C.

HRESULT FDConfirmFrame() – mark copied video and audio data as ready to play. FDExt board can play it right now. If current time of the board more than frame number specified in method FDAllocateVideoFrame, then method FDConfirmFrame return S_FALSE.

HRESULT FDGetBoardTime([out] uint64* pFrameNumber, [out] uint64* pFrameTime) – return current frame number and current system time for the board.

HRESULT FDGetFifoStats([out] unsigned long* pMaxFifoSizeInFrames, [out] unsigned long* pFifoSizeInFrames, [out] uint64* pFrameNumber, [out] uint64* pFrameTime) – return info about playback state. Parameter "pMaxFifoSizeInFrames" return size of output fifo in frames. Parameter "pFifoSizeInFrames" return number of filled frames into the output fifo. Parameter "pFrameNumber" return current frame number. Parameter "pFrameTime" return current system time.

Notes:

- Structure CBoardVideoParams has follow fields:

long m_DX 	frame width (in pixel)

long m_DY 	frame height

double m_FrameRate 	frame rate

double m_UpperFieldTimerShift 	delay from frame start to upper field start

double m_LowerFieldTimerShift 	delay from frame start to lower field start

long m_UpperFirst 	1 if upper field first, 0 if lower field first

Field m_UpperFieldTimerShift or m_LowerFieldTimerShift must be zero.

- Structure CBoardSoundParams has follow fields:

long m_Valid 	not used

long m_SampleRate 	sound ratio (48000)

long m_Channels 	number of audio channels (4 as usual)

double m_FrameRate 	frame rate

- To add video and audio data you should make follow steps:

- Allocate space into output fifo by call FDAllocateVideoFrame.

- Add video data by call FDWriteVideoFrameData or double call FDWriteVideoFieldData. Also you can and audio data by call FDWriteSoundData.

- Mark "all done" by call FDConfirmFrame.

You should add frame with frame number in the nearest future like current frame + 2. Thus you should call FDGetBoardTime to check current time.

## IFDSoundOut

Interface IFDSoundOut allow playback audio on FDExt board output.

HRESULT FDGetSoundParams([out] CBoardSoundParams* pSoundParams) – return format of audio data. Programs have to add audio data with specified ratio only.

HRESULT FDGetPreferredFormats([out] CFDCaptureSoundFormat* pDataSoundFormat) – return preferred audio sample format. It is native format of FDExt board.

HRESULT FDWriteSoundData([in] uint64 FrameNumber, [in] unsigned char* Buffer, [in] unsigned long cb, [in] CFDCaptureSoundFormat DataFormat, [in] unsigned long ChannelMask) – copy audio data for specified video frame into output fifo. Duration of the audio data should correspond video duration with one sample accuracy (+/- one sample). Parameter "Buffer" specifies pointer of audio data. Parameter "cb" specifies size (in bytes) of audio data. Parameter "DataFormat" specifies sample format. Parameter "ChannelMask" specifies mask for output audio data. For example first language output should have mask 0x03, second language output should have mask 0xC0.

HRESULT FDGetBoardTime([out] uint64* pFrameNumber, [out] uint64* pFrameTime) – return current frame number and current system time for the board.

HRESULT FDGetFifoStats([out] unsigned long* pMaxFifoSizeInFrames, [out] uint64* pFrameNumber, [out] uint64* pFrameTime) – return info about playback state. Parameter "pMaxFifoSizeInFrames" return size of output fifo in frames. Parameter "pFrameNumber" return current frame number. Parameter "pFrameTime" return current system time.

# Appendix

## Example: select video player

Strongly recommended to check result of each operation (HRESULT) with the help macro FAILED and to stop execution if the call of any method has returned any error.

This is a procedure to fill list of available players:

HRESULT FillList (CComboBox DeviceList)

{

// create object FDExtMgr

CComPtr<FDExtSrvdLib::IFDExtMgr> pIMgr;

HRESULT hr = pIMgr.CoCreateInstance(FDExtSrvdLib::CLSID_FDExtMgr);

ATLASSERT(hr==S_OK);

// scan all valid video players

long nIndex = 0;

FDExtSrvdLib::CFDObjectInfo info;

while( S_OK == pIMgr->get_NextValidFDObjectInfo(&nIndex,
 FDExtSrvdLib::FDObjType_VideoOut, &info) )

{

// use the player with index nIndex-1

CComBSTR bstr;

hr = pIMgr->GetObjectFullName(&info, &bstr);

ATLASSERT(hr==S_OK);

DeviceList.AddString(CString(bstr));

}

return S_OK;

}

This is a procedure to open player with specified name:

HRESULT GetPlayer (CString szName, CComPtr<FDExtSrvdLib::IFDVideoOut> &pIPlayer)

{

// create object FDExtMgr

CComPtr<FDExtSrvdLib::IFDExtMgr> pIMgr;

HRESULT hr = pIMgr.CoCreateInstance(FDExtSrvdLib::CLSID_FDExtMgr);

ATLASSERT(hr==S_OK);

// create video player

CComBSTR bstr(szName);

CComPtr<IUnknown> pIUnk;

hr = pIMgr->get_FDObjectByName(bstr,
 FDExtSrvdLib::FDObjType_VideoOut, & pIUnk);

ATLASSERT(hr==S_OK);

ATLASSERT(pIUnk != NULL);

hr = pIUnk->QueryInterface(&pIPlayer);

ATLASSERT(hr==S_OK);

ATLASSERT(pIPlayer != NULL);

return S_OK;

}