Forward Splicer
Draft description of internal interfaces

Forward Splicer solution:

Splicer object receive input SPTS, demux it to separate elementary streams and write each stream into FIFO. Spicer scans SCTE35 marks on input and remembers OUT & IN points. Also, Splicer object read TS file with insertion and demux it to separate elementary streams (ES). Splicer object mux output SPTS from elementary streams. It may be streams from FIFO or from file with insertion. On OUT point Splicer select nearest switch point per every input ES and switch to file’s ES. On IN point Splicer select nearest switch point per every input ES and file’s ES and return back to input. On each OUT and IN point Splicer remembers time delay for next insertion.

Splicer state machine:

Encoder for Forward Splicer

Encoder for Forward Splicer allow convert insertion block (list of movies) into one ready to insert SPTS file. Before start Encoder should be configured by video/audio and other settings. Each setting is XML file. Then you need add list of movie files (file name, MarkIn and MarkOut) and specifies output TS file name. During encoding you can obtain progress and encoding state. Encoder support video encoding to AVC or MPEG2 and audio encoding to AAC or MPEG1/2.

Schedule for Forward Splicer

Forward Splicer support SCTE-118. It downloads schedule and movies from FTP server and upload reports to the FTP server. Softlab has own SCTE-118 server for easy movie and schedule management: