# Low-level FDExt SDK

This document describes low-level SDK for FDExt boards (FD322, FD422 and FD842).

You should insert the following code into "stdafx.h" file (for C++ programs):

//FDExt

#import "libid:C03DE31D-62BA-4114-AB08-06F493FB98FF" named_guids raw_interfaces_only

using namespace FDExtSrvdLib;

## Interfaces and objects of FDExt SDK

CLSID_FDExtMgr :

interface IFDExtMgr;

interface IFDBoard;

interface IFDCaptureVideo;

interface IFDCaptureVideoEx;

interface IFDCaptureASI;

interface IFDASIOut;

interface IFDASIOutSettings;

interface IFDWatchDog;

interface IFDVideoOut;

interface IFDCaptureState;

interface IFDCaptureAudio;

interface IFDCaptureLng;

interface IFDExtMgrLng;

interface IFDExtMgrAVObjectsInfo;

typedef enum {

FDCaptureVideoFormat_BGRA = 0,

FDCaptureVideoFormat_BGR = 1,

FDCaptureVideoFormat_UYVY = 2,

FDCaptureVideoFormat_YUY2 = 3

} CFDCaptureVideoFormat;

typedef enum {

FDCaptureSoundFormat_16bit = 0,

FDCaptureSoundFormat_24bit = 1,

FDCaptureSoundFormat_IEEEFloat = 2

} CFDCaptureSoundFormat;

typedef struct tagCBoardVideoParams {

long m_DX;

long m_DY;

double m_FrameRate;

double m_UpperFieldTimerShift;

double m_LowerFieldTimerShift;

long m_UpperFirst;

} CBoardVideoParams;

typedef struct tagCBoardSoundParams {

long m_Valid;

long m_SampleRate;

long m_Channels;

double m_FrameRate;

} CBoardSoundParams;

typedef enum {

FDObjType_Any = 0,

FDObjType_Board = 1,

FDObjType_VideoSrc = 2,

FDObjType_VideoOut = 3,

FDObjType_TitleTarget = 4,

FDObjType_ASISrc = 5,

FDObjType_RawASISrc = 6,

FDObjType_ASIOut = 7,

FDObjType_RawASIOut = 8,

FDObjType_VideoMixSrc = 9,

FDObjType_VideoPreviewSrc = 10,

FDObjType_VideoStereoSrc = 11,

FDObjType_WatchDog = 12,

FDObjType_GenlockSrc = 13,

FDObjType_ColorKeyer = 14,

FDObjType_AudioSrc = 15,

FDObjType_AudioMixSrc = 16,

FDObjType_Total = 17

} CFDObjType;

typedef struct tagCFDObjectInfo {

long m_Valid;

CFDObjType m_FDObjType;

unsigned short m_FDObjRegName[128];

unsigned short m_FDObjName[128];

unsigned short m_FDObjFriendlyName[128];

long m_BoardIndex;

long m_IndexInBoard;

long m_ObjTypeChannel;

long m_SoundChannels;

unsigned short m_BoardName[128];

unsigned short m_BoardType[128];

} CFDObjectInfo;

typedef enum CWSSFlags

{

frame_wss_16_9 = 1,

} CWSSFlags;

typedef enum FDExtWatchDogState {

FDExtWatchDog_None	=	-1,

FDExtWatchDog_Expired	=	0,

FDExtWatchDog_Active	=	1

} FDExtWatchDogState;

## IFDExtMgr interface

It is a basic interface. It allows to access all other objects of low-level FDExt SDK.

coclass: CLSID_FDExtMgr

interface IFDExtMgr : IUnknown {

HRESULT ConnectRemoteHost ([in] BSTR HostName);

HRESULT get_BoardsCount([out, retval] long* pVal);

HRESULT get_BoardByIndex([in] long Index, [out, retval] IUnknown** ppUnk);

HRESULT get_FDObjectsCount([out, retval] long* pVal);

HRESULT get_FDObjectByIndex([in] long Index, [in] CFDObjType FDObjType,

[out, retval] IUnknown** ppUnk);

HRESULT get_FDObjectInfoByIndex([in] long Index, [out, retval] CFDObjectInfo* pFDObjectInfo);

HRESULT get_FDObjectByName([in] BSTR Name, [in] CFDObjType FDObjType,

[out, retval] IUnknown** ppUnk);

HRESULT get_NextValidFDObjectInfo([in, out] long* pNextIndex, [in] CFDObjType FDObjType,

[out, retval] CFDObjectInfo* pFDObjectInfo);

HRESULT TranslateFDObjectInfo([in] unsigned long LocaleID,

[in, out] CFDObjectInfo* pFDObjectInfo);

HRESULT GetTitleEngineInfo([in] CFDObjectInfo * pFDObjectInfo,

[out] GUID * pTitleEngineClsid, [out] long* pTitleEngineIndex);

HRESULT get_FDRemoteObjectByIndex([in] long Index, [in] CFDObjType FDObjType,

[out, retval] IUnknown** ppUnk);

HRESULT get_FDRemoteObjectByName([in] BSTR Name, [in] CFDObjType FDObjType,

[out, retval] IUnknown** ppUnk);

HRESULT GetObjectFullName([in] CFDObjectInfo* pFDObjectInfo, [out] BSTR* pName);

};

HRESULT ConnectRemoteHost ([in] BSTR HostName);

Connect created object with FDExt server on remote computer with network name “HostName”. Don’t forget to initialize DCOM and security correctly in your code before connecting.

If this function isn't called, created object is connected to local server (on local host).

HRESULT get_BoardsCount([out, retval] long* pVal);

Returns total boards number in the system – all FD422, FD322, FD842 and “virtual” boards.

HRESULT get_BoardByIndex([in] long Index, [out, retval] IUnknown** ppUnk);

Returns COM object for board with specified index (starting from zero). Object “ppUnk” provides the IFDBoard interface (see below).

HRESULT get_FDObjectsCount([out, retval] long* pVal);

Returns total number of different COM objects for all boards (including “virtual” boards). They are boards, capture and playback objects, title engines and other. Object may be valid or not – use object information to check it.

HRESULT get_FDObjectByIndex([in] long Index, [in] CFDObjType FDObjType,

[out, retval] IUnknown** ppUnk);

Returns COM object for object with specified index (starting from zero). Object type must be the same as specified in “FDObjType”. Object “ppUnk” provides interface corresponding to the type:

typedef enum {

FDObjType_Any,		// any type – don’t use it

FDObjType_Board,		// board

FDObjType_VideoSrc,		// capture video/audio from board input

FDObjType_VideoOut,		// playback video/audio to board output

FDObjType_TitleTarget,	// title engine on board output

FDObjType_ASISrc,		// capture from ASI input

FDObjType_RawASISrc,	// capture “raw” data from ASI input

FDObjType_ASIOut,		// playback to ASI output

FDObjType_RawASIOut,	// playback “raw” data to ASI output

FDObjType_VideoMixSrc,	// capture video/audio from board output

FDObjType_VideoPreviewSrc,// capture preview video/audio (input & output)

FDObjType_VideoStereoSrc, // capture double video input as stereo picture

FDObjType_WatchDog,		// WatchDog

FDObjType_GenlockSrc,     // (not used)

FDObjType_ColorKeyer,     // (not used)

FDObjType_AudioSrc,		// capture audio from board input

FDObjType_AudioMixSrc,    // capture audio from board output

} CFDObjType;

Notes: 1. Object with type “FDObjType_VideoPreviewSrc” can capture video without valid license for capturing from input/output. Use interface IFDCaptureVideo or IFDCaptureVideoEx to capture. The object skips frames regularly.

2. On creation of capture object FDExt server starts capturing process. It loads the system (CPU and memory usage, PCI-Express bus usage). FDExt server stops capturing process with some delay after release of capture object. Therefore it is recommended to create capture object only when it is really needed to work.

3. Audio capture object with types “FDObjType_AudioSrc” or “FDObjType_AudioMixSrc” correspond to video/audio capture objects with type “FDObjType_VideoSrc” or “FDObjType_VideoMixSrc”, but it can capture audio only. It has the same friendly name as video object.

4. Object with type “WatchDog” allows to monitor system viability. It has the same name as corresponding title engine (or board output).

HRESULT get_FDObjectInfoByIndex([in] long Index, [out, retval] CFDObjectInfo* pFDObjectInfo);

Returns description for object with specified index (all text is WCHAR):

typedef struct tagCFDObjectInfo {

long m_Valid;			// valid if > 0

CFDObjType m_FDObjType;	// object type (see above)

unsigned short m_FDObjRegName[128]; // unique object name (see below)

unsigned short m_FDObjName[128]; // object name

unsigned short m_FDObjFriendlyName[128]; // user-friendly name of object

long m_BoardIndex;		// board index

long m_IndexInBoard;	// index for the same board objects

long m_ObjTypeChannel;	// index for the same type and board objects

long m_SoundChannels;	// sound mono channels number (typically 4)

unsigned short m_BoardName[128]; // board name

unsigned short m_BoardType[128]; // name of board type (e.g. “FD422”)

} CFDObjectInfo;

Notes: 1. The m_Valid field contains zero value if the object is disabled in current board configuration or by user settings. For example, SDI capture object is not valid on ASI capture configuration. The object can be used if the value is greater than zero. If the object has no license the value is negative.

2. The m_FDObjRegName field is unique for the board. It will not be changed on adding of new board. It will not be translated (see below). Use two fields “m_BoardName” and “m_FDObjRegName” for global identification of FDExt objects. For example, your application can store them and then enumerate all objects until object with stored “m_BoardName” and “m_FDObjRegName” names is found.

ATTENTION: it is better to use unique object name obtained by method “GetObjectFullName” (see below). See also section «Examples», subsection «Saving/Loading selected device in program settings».

HRESULT get_FDObjectByName([in] BSTR Name, [in] CFDObjType FDObjType,

[out, retval] IUnknown** ppUnk);

Returns COM object for FDExt object with specified name. Object type must be the same as specified in “FDObjType”. Returned “ppUnk” object provides interface corresponding to the type. The name consists of board name and object name with symbol “/” between them. For example: «FD422 Board 1/Output» corresponds to video output of first FD422 board.

HRESULT get_NextValidFDObjectInfo([in, out] long* pNextIndex, [in] CFDObjType FDObjType,

[out, retval] CFDObjectInfo* pFDObjectInfo);

Allows to enumerate all valid objects with specified object type. Method returns information for next valid object. You have to initialize “long” variable by zero and use its address as “pNextIndex”. Then call method while it returns S_OK. Index of the object is “*pNextIndex – 1”.

HRESULT TranslateFDObjectInfo([in] unsigned long LocaleID,

[in, out] CFDObjectInfo* pFDObjectInfo);

The method translates names in specified object description structure to specified language. Now it supports Russian localization only (LocaleID=0x0419).

HRESULT GetTitleEngineInfo([in] CFDObjectInfo * pFDObjectInfo,

[out] GUID * pTitleEngineClsid, [out] long* pTitleEngineIndex);

Returns title engine type and index to “*pTitleEngineClsid” and “*pTitleEngineIndex” for specified object description. Object type must be title engine only. Otherwise the method returns E_INVALIDARG.

HRESULT get_FDRemoteObjectByIndex([in] long Index, [in] CFDObjType FDObjType,

[out, retval] IUnknown** ppUnk);

Do not use it.

HRESULT get_FDRemoteObjectByName([in] BSTR Name, [in] CFDObjType FDObjType,

[out, retval] IUnknown** ppUnk);

Do not use it.

HRESULT GetObjectFullName([in] CFDObjectInfo* pFDObjectInfo, [out] BSTR* pName);

Returns global unique name for specified object description. It is recommended to use this name to save/restore selection of the object. The name can be used as input parameter for method “get_FDObjectByName”.

## IFDBoard interface

The interface is available for FDExt object with “FDObjType_Board” type. Object can be obtained by IFDExtMgr interface (see above).

interface IFDBoard : IUnknown {

HRESULT get_BoardName([out, retval] BSTR* pName);

HRESULT get_BoardTime([out] uint64* pFrameNumber, [out] uint64* pFrameFileTime);

HRESULT get_BoardVideoParams([out] CBoardVideoParams * pVideoParams);

HRESULT get_BoardSoundParams([out] CBoardSoundParams * pSoundParams);

HRESULT get_BoardFrameRate([out] double* pFrameRate);

HRESULT get_BoardFieldEventName([out] BSTR* pEventName);

};

HRESULT get_BoardName([out, retval] BSTR* pName);

Returns board name.

HRESULT get_BoardTime([out] uint64* pFrameNumber, [out] uint64* pFrameFileTime);

Returns information about current board time. Parameter “pFrameNumber” returns number of current playback frame. Parameter “pFrameFileTime” returns system time (GMT) as FILETIME for the frame.

HRESULT get_BoardVideoParams([out] CBoardVideoParams * pVideoParams);

Returns video parameters of the board. See description of the structure CBoardVideoParams below (interface IFDCaptureVideo).

HRESULT get_BoardSoundParams([out] CBoardSoundParams * pSoundParams);

Returns audio parameters of the board. See description of the structure CBoardSoundParams below (interface IFDCaptureVideo).

HRESULT get_BoardFrameRate([out] double* pFrameRate);

Returns board frame rate (frames per seconds).

HRESULT get_BoardFieldEventName([out] BSTR* pEventName);

Returns name of system synchronization object (event) for frame interrupts. FDExt calls PulseEvent for the event on every frame interrupt.

## IFDCaptureVideo interface

- The interface is available for FDExt objects with “FDObjType_VideoSrc”, “FDObjType_VideoMixSrc” or “FDObjType_VideoPreviewSrc” types. Such object can be obtained by IFDExtMgr interface (see above). It allows to capture video and audio from board input or output. The object must be valid.

interface IFDCaptureVideo : IUnknown {

HRESULT FDReadVideoFrameData([out] unsigned char* Buffer, [in] unsigned long cb,

[in, out] uint64* pFrameNumber, [out] uint64* pWSSFlags,

[in] CFDCaptureVideoFormat DataFormat);

HRESULT FDReadVideoFieldData([out] unsigned char* Buffer, [in] unsigned long cb,

[in, out] uint64* pFrameNumber, [out] uint64* pWSSFlags,

[in] CFDCaptureVideoFormat DataFormat, [in] char UpperField);

HRESULT FDReadSoundData([out] unsigned char* Buffer, [in] unsigned long cb,

[in, out] uint64* pFrameNumber, [out] unsigned long* prtSize,

[in] CFDCaptureSoundFormat DataFormat, [in] unsigned long ChannelMask);

HRESULT GetSoundLevels([out] double* pLevels, [in] unsigned long LevelsNumber,

[out] uint64* pFrameNumber, [in] unsigned long ChannelMask);

HRESULT FDReadWSSData([out] uint64* pWSSFlags);

HRESULT FDGetVideoTime([out] uint64* pFrameNumber, [out] uint64* pFrameTime);

HRESULT FDGetVideoParams([out] CBoardVideoParams* pVideoParams);

HRESULT FDGetSoundParams([out] CBoardSoundParams* pSoundParams);

};

HRESULT FDReadVideoFrameData([out] unsigned char* Buffer, [in] unsigned long cb,

[in, out] uint64* pFrameNumber, [out] uint64* pWSSFlags,

[in] CFDCaptureVideoFormat DataFormat);

Copies captured frame image into specified “Buffer”. Parameter “cb” specifies buffer size (in bytes). Parameter “pFrameNumber” specifies pointer to next frame number which should be captured. The parameter returns real frame number of captured frame. If returned frame number differs from requested frame number then some frames were dropped. You can obtain current frame number of the board by calling “FDGetVideoTime” (see below). Description of parameter “pWSSFlags” see below (method “FDReadWSSData”).

Parameter “DataFormat” specifies format of returned video data. It can be:

typedef enum {

FDCaptureVideoFormat_BGRA,	// BGRA (32 bit per pixel)

FDCaptureVideoFormat_BGR,		// BGR  (24 bit per pixel)

FDCaptureVideoFormat_UYVY,	// UYVY (16 bit per pixel)

FDCaptureVideoFormat_YUY2		// YUY2 (16 bit per pixel)

} CFDCaptureVideoFormat;

It is recommended to use UYVY or YUY2 formats.

ATTENTION: the method always captures upper field first and lower field next on default settings of the boards.

HRESULT FDReadVideoFieldData([out] unsigned char* Buffer, [in] unsigned long cb,

[in, out] uint64* pFrameNumber, [out] uint64* pWSSFlags,

[in] CFDCaptureVideoFormat DataFormat, [in] char UpperField);

Copies captured image of specified field (upper or lower) into specified “Buffer”. Parameter “cb” specifies buffer size (in bytes). Parameter “pFrameNumber” specifies pointer to next frame number which should be captured. The parameter returns real frame number of captured frame. If returned frame number differs from requested frame number then some frames were dropped. You can obtain current frame number of the board by calling “FDGetVideoTime” (see below). Description of parameter “pWSSFlags” see below (method “FDReadWSSData”).

Parameter DataFormat specifies format of returned video data (see above).

If “UpperField” is zero then lower field is captured. Otherwise upper field is captured. Buffer will contain field image as solid picture (without empty lines in the picture).

HRESULT FDReadSoundData([out] unsigned char* Buffer, [in] unsigned long cb,

[in, out] uint64* pFrameNumber, [out] unsigned long* prtSize,

[in] CFDCaptureSoundFormat DataFormat, [in] unsigned long ChannelMask);

Copied captured audio data for specified frame into specified “Buffer”. Parameter “cb” specifies buffer size (in bytes). Parameter “prtSize” returns size of captured audio data (in bytes). Parameter “pFrameNumber” specifies pointer to next frame number which should be captured. The parameter returns real frame number of captured frame. If returned frame number differs from requested frame number then some frames were dropped. You can obtain current frame number of the board by calling “FDGetVideoTime” (see below).

Parameter “DataFormat” specifies format of returned audio data. It can be:

typedef enum {

FDCaptureSoundFormat_16bit,	// 16-bit audio

FDCaptureSoundFormat_24bit,	// 24-bit audio

FDCaptureSoundFormat_IEEEFloat, // normalized float audio (-1.0 ... 1.0)

} CFDCaptureSoundFormat;

Parameter “ChannelMask” specifies bit mask of audio channels to capture. To capture channel number N set corresponding bit to “1” in the mask.

Audio channels go in the following order:

L1 (left channel of first audio stereo stream)

R1 (right channel of first audio stereo stream)

L2 (left channel of second audio stereo stream)

R2 (right channel of second audio stereo stream)

...

Total number of audio channels is specified in the m_Channels field of the structure CBoardSoundParams (see below description of FDGetSoundParams). Normally SDI signal contains 4 audio channels.

HRESULT GetSoundLevels([out] double* pLevels, [in] unsigned long LevelsNumber,

[out] uint64* pFrameNumber, [in] unsigned long ChannelMask);

Copies set of audio levels into specified buffer “pLevels”. You can use it to display current audio levels by audio volume indicators (meters). Parameter “LevelsNumber” specifies number of levels to capture. Each level is a “double” value between 0.0 and 1.0 in linear scale (value 0.0 corresponds to silence, value 1.0 corresponds to maximum volume).

Parameter “pFrameNumber” returns frame number of copied audio levels. If it is the same number as in last call then you need not redraw audio levels.

Parameter “ChannelMask” specifies bit mask of audio channels to capture (see description above).

HRESULT FDReadWSSData([out] uint64* pWSSFlags);

Returns current state of WSS flag in video signal. If value is “frame_wss_16_9” then frame has aspect ratio is 16:9. Otherwise frame has aspect ratio 4:3.

HRESULT FDGetVideoTime([out] uint64* pFrameNumber, [out] uint64* pFrameTime);

Returns frame number of newest frame in the board capture FIFO. Parameter “pFrameNumber” returns number of the frame. Parameter “pFrameFileTime” returns system time (GMT) as FILETIME for the frame.

HRESULT FDGetVideoParams([out] CBoardVideoParams* pVideoParams);

Returns video parameters of capture object:

typedef struct tagCBoardVideoParams {

long m_DX;				// frame image width (in pixels)

long m_DY;				// frame image height (in pixels)

double m_FrameRate;			// frame rate (frames per second)

double m_UpperFieldTimerShift;	// upper field delay (in seconds)

double m_LowerFieldTimerShift;	// lower field delay (in seconds)

long m_UpperFirst;			// non zero if upper field first

} CBoardVideoParams;

If upper field delay is zero and lower field delay is positive then video is interlaced with field order Upper Field First. If lower field delay is zero and upper field delay is positive then video is interlaced with field order Lower Field First. If both delays are zero the video is progressive.

HRESULT FDGetSoundParams([out] CBoardSoundParams* pSoundParams);

Returns audio parameters of capture object:

typedef struct tagCBoardSoundParams {

long m_Valid;				// audio is valid if > 0

long m_SampleRate;			// audio sample rate (samples per second)

long m_Channels;			// channels number (mono channels)

double m_FrameRate;			// video frame rate (frames per second)

} CBoardSoundParams;

If audio is not valid then method “FDReadSoundData” will return zero size of captured audio data in parameter “prtSize”. Thus you can’t capture audio.

## IFDCaptureVideoEx interface

This interface is almost the same as “IFDCaptureVideo” and has additional methods:

interface IFDCaptureVideo : IUnknown {

HRESULT FDReadVideoFrameDataEx ([out,size_is(cb)] BYTE* Buffer, [in] ULONG cb, [in] LONG stride,

[in] LONG x, [in] LONG y, [in] LONG dx, [in] LONG dy,

[in, out] ULONGLONG* pFrameNumber, [out] ULONGLONG* pWSSFlags,

[in] CFDCaptureVideoFormat DataFormat);

HRESULT FDReadVideoFieldDataEx ([out,size_is(cb)] BYTE* Buffer, [in] ULONG cb, [in] LONG stride,

[in] LONG x, [in] LONG y, [in] LONG dx, [in] LONG dy,

[in, out] ULONGLONG* pFrameNumber, [out] ULONGLONG* pWSSFlags,

[in] CFDCaptureVideoFormat DataFormat, [in] BOOLEAN UpperField);

HRESULT FDReadVideoFrameData([out] unsigned char* Buffer, [in] unsigned long cb,

[in, out] uint64* pFrameNumber, [out] uint64* pWSSFlags,

[in] CFDCaptureVideoFormat DataFormat);

HRESULT FDReadVideoFieldData([out] unsigned char* Buffer, [in] unsigned long cb,

[in, out] uint64* pFrameNumber, [out] uint64* pWSSFlags,

[in] CFDCaptureVideoFormat DataFormat, [in] char UpperField);

HRESULT FDReadSoundData([out] unsigned char* Buffer, [in] unsigned long cb,

[in, out] uint64* pFrameNumber, [out] unsigned long* prtSize,

[in] CFDCaptureSoundFormat DataFormat, [in] unsigned long ChannelMask);

HRESULT GetSoundLevels([out] double* pLevels, [in] unsigned long LevelsNumber,

[out] uint64* pFrameNumber, [in] unsigned long ChannelMask);

HRESULT FDReadWSSData([out] uint64* pWSSFlags);

HRESULT FDGetVideoTime([out] uint64* pFrameNumber, [out] uint64* pFrameTime);

HRESULT FDGetVideoParams([out] CBoardVideoParams* pVideoParams);

HRESULT FDGetSoundParams([out] CBoardSoundParams* pSoundParams);

HRESULT FDGetFifoStats([out] ULONG* pMaxFifoSizeInFrames, [out] ULONG* pFifoSizeInFrames,

[out] ULONGLONG* pNewestFrameNumber, [out] ULONGLONG* pFrameTime);

};

HRESULT FDReadVideoFrameDataEx ([out,size_is(cb)] BYTE* Buffer, [in] ULONG cb,

[in] LONG stride, [in] LONG x, [in] LONG y, [in] LONG dx, [in] LONG dy,

[in, out] ULONGLONG* pFrameNumber, [out] ULONGLONG* pWSSFlags,

[in] CFDCaptureVideoFormat DataFormat);

Copies captured frame image into specified buffer as method “FDReadVideoFrameData”. Additional parameters:

- “stride” – offset in bytes from one line to another in the picture buffer. It may be negative to flip picture vertically (in any case parameter “Buffer” specifies begin of picture buffer).

- “x”, “y”, “dx”, “dy” specifies “area of interest” – only specified rectangle part will be copied to picture buffer.

HRESULT FDReadVideoFieldDataEx ([out,size_is(cb)] BYTE* Buffer, [in] ULONG cb,

[in] LONG stride, [in] LONG x, [in] LONG y, [in] LONG dx, [in] LONG dy,

[in, out] ULONGLONG* pFrameNumber, [out] ULONGLONG* pWSSFlags,

[in] CFDCaptureVideoFormat DataFormat, [in] BOOLEAN UpperField);

Copied captured field image into specified buffer as method “FDReadVideoFieldData”. Additional parameters:

- “stride” – offset in bytes from one line to another in the picture buffer. It may be negative to flip picture vertically (in any case parameter “Buffer” specifies begin of picture buffer).

- “x”, “y”, “dx”, “dy” specifies “area of interest” – only specified rectangle part will be copied to picture buffer.

HRESULT FDGetFifoStats([out] ULONG* pMaxFifoSizeInFrames,

[out] ULONG* pFifoSizeInFrames, [out] ULONGLONG* pNewestFrameNumber,

[out] ULONGLONG* pFrameTime);

Returns current state of capture FIFO. Parameter “pMaxFifoSizeInFrames” returns FIFO size – maximum number of frames that can be stored in FIFO together. Parameter “pFifoSizeInFrames” returns current number of frames in FIFO. Parameter “pNewestFrameNumber” returns number of newest frame in FIFO. Parameter “pFrameTime” returns system time (GMT) as FILETIME for the newest frame in FIFO.

## IFDCaptureASI interface

This interface allows to capture ASI data.

interface IFDCaptureASI : IUnknown {

HRESULT FDGetBoardTime([out] uint64* pFrameNumber, [out] uint64* pFrameFileTime);

HRESULT FDGetFrameDuration([out] unsigned long* pFrameDurationIn27MgzClocks);

HRESULT FDGetFrameBuffer([in] unsigned long cb, [out] unsigned char** ppData);

HRESULT FDConfirmFrameBuffer([in] unsigned long cb, [in] uint64 FrameNumber);

};

HRESULT FDGetBoardTime([out] uint64* pFrameNumber, [out] uint64* pFrameFileTime);

...

HRESULT FDGetFrameDuration([out] unsigned long* pFrameDurationIn27MgzClocks);

...

HRESULT FDGetFrameBuffer([in] unsigned long cb, [out] unsigned char** ppData);

...

HRESULT FDConfirmFrameBuffer([in] unsigned long cb, [in] uint64 FrameNumber);

...

## IFDASIOutSettings interface

- This interface allows to change ASI settings.

interface IFDASIOutSettings : IUnknown {

HRESULT SetASIPacketLength([in] unsigned long cb);

HRESULT EnablePacketSpread([in] unsigned long Enable);

};

HRESULT SetASIPacketLength([in] unsigned long cb);

...

HRESULT EnablePacketSpread([in] unsigned long Enable);

...

## IFDCaptureState interface

- This interface is available together with “IFDCaptureVideo” interface.

interface IFDCaptureState : IUnknown {

HRESULT GetSelectedInputName([out] BSTR* pCurrentInputName);

HRESULT GetInputState([out] long* pSignalExist, [out] long* pDataExist,

[out] double* pFrameRate, [out] long* pHasWSS);

HRESULT GetLastValidFrameInfo([out] int64* pSystemTime);

};

HRESULT GetSelectedInputName([out] BSTR* pCurrentInputName);

Returns name of current video source for the capture object. This method is useful for FD322 board – it has many analog inputs. For other boards the method returns empty text string.

HRESULT GetInputState([out] long* pSignalExist, [out] long* pDataExist, [out] double* pFrameRate, [out] long* pHasWSS);

Returns some flags for corresponding board input.

Parameter “pSignalExist” returns zero if the board can’t detect carrier signal on corresponding board input. In this case client application can show warning message “No signal”.

Parameter “pDataExist” returns zero if the board can’t recognize video signal on corresponding board input. In this case client application can show warning message “No data” or “Bad signal”.

Parameter “pFrameRate” returns “real” frame rate on board input which is calculated by system clock. It is not “ideal” frame rate from board settings. You can compare real frame rate with ideal frame rate and show warning message if difference is too big.

Parameter “pHasWSS” returns non-zero value if input video has aspect ratio 16:9. It returns zero for 4:3 aspect ratio of input video.

HRESULT GetLastValidFrameInfo([out] int64* pSystemTime);

Returns system time (GMT) as FILETIME for last detected valid video frame. You can compare the time with current time to detect problem on video input.

## IFDCaptureAudio interface

- The interface is available for FDExt objects with “FDObjType_AudioSrc” or “FDObjType_AudioMixSrc” types. Object can be obtained by IFDExtMgr interface (see above). It allows to capture audio from board input or output. Object must be valid.

The interface methods are the same as IFDCaptureVideo methods.

interface IFDCaptureAudio : IUnknown {

HRESULT FDReadSoundData([out] unsigned char* Buffer, [in] unsigned long cb,

[in, out] uint64* pFrameNumber, [out] unsigned long* prtSize,

[in] CFDCaptureSoundFormat DataFormat, [in] unsigned long ChannelMask);

HRESULT GetSoundLevels([out] double* pLevels, [in] unsigned long LevelsNumber,

[out] uint64* pFrameNumber, [in] unsigned long ChannelMask);

HRESULT FDGetVideoTime([out] uint64* pFrameNumber, [out] uint64* pFrameTime);

HRESULT FDGetSoundParams([out] CBoardSoundParams* pSoundParams);

HRESULT FDGetFifoStats([out] unsigned long* pMaxFifoSizeInFrames,

[out] unsigned long* pFifoSizeInFrames, [out] uint64* pNewestFrameNumber,

[out] uint64* pFrameTime);

};

HRESULT FDReadSoundData([out] unsigned char* Buffer, [in] unsigned long cb,

[in, out] uint64* pFrameNumber, [out] unsigned long* prtSize,

[in] CFDCaptureSoundFormat DataFormat, [in] unsigned long ChannelMask);

Copied captured audio data for specified frame into specified “Buffer”. Parameter “cb” specifies buffer size (in bytes). Parameter “prtSize” returns size of captured audio data (in bytes). Parameter “pFrameNumber” specifies pointer to next frame number which should be captured. The parameter returns real frame number of captured frame. If returned frame number differs from requested frame number then some frames were dropped. You can obtain current frame number of the board by calling “FDGetVideoTime” (see below).

Parameter “DataFormat” specifies format of returned audio data. It can be:

typedef enum {

FDCaptureSoundFormat_16bit,	// 16-bit audio

FDCaptureSoundFormat_24bit,	// 24-bit audio

FDCaptureSoundFormat_IEEEFloat, // normalized float audio (-1.0 ... 1.0)

} CFDCaptureSoundFormat;

Parameter “ChannelMask” specifies bit mask of audio channels to capture. To capture channel number N set corresponding bit to “1” in the mask.

Audio channels go in the following order:

L1 (left channel of first audio stereo stream)

R1 (right channel of first audio stereo stream)

L2 (left channel of second audio stereo stream)

R2 (right channel of second audio stereo stream)

...

Total number of audio channels is specified in the m_Channels field of the structure CBoardSoundParams (see below description of FDGetSoundParams). Normally SDI signal contains 4 audio channels.

HRESULT GetSoundLevels([out] double* pLevels, [in] unsigned long LevelsNumber,

[out] uint64* pFrameNumber, [in] unsigned long ChannelMask);

Copies set of audio levels into specified buffer “pLevels”. You can use it to display current audio levels by audio volume indicators (meters). Parameter “LevelsNumber” specifies number of levels to capture. Each level is a “double” value between 0.0 and 1.0 in linear scale (value 0.0 corresponds to silence, value 1.0 corresponds to maximum volume).

Parameter “pFrameNumber” returns frame number of copied audio levels. If it is the same number as in last call then you need not redraw audio levels.

Parameter “ChannelMask” specifies bit mask of audio channels to capture (see description above).

HRESULT FDGetVideoTime([out] uint64* pFrameNumber, [out] uint64* pFrameTime);

Returns frame number of newest frame in the board capture FIFO. Parameter “pFrameNumber” returns number of the frame. Parameter “pFrameFileTime” returns system time (GMT) as FILETIME for the frame.

HRESULT FDGetSoundParams([out] CBoardSoundParams* pSoundParams);

Returns audio parameters of capture object:

typedef struct tagCBoardSoundParams {

long m_Valid;				// audio is valid if > 0

long m_SampleRate;			// audio sample rate (samples per second)

long m_Channels;			// channels number (mono channels)

double m_FrameRate;			// video frame rate (frames per second)

} CBoardSoundParams;

HRESULT FDGetFifoStats([out] ULONG* pMaxFifoSizeInFrames,

[out] ULONG* pFifoSizeInFrames, [out] ULONGLONG* pNewestFrameNumber,

[out] ULONGLONG* pFrameTime);

Returns current state of capture FIFO. Parameter “pMaxFifoSizeInFrames” returns FIFO size – maximum number of frames that can be stored in FIFO together. Parameter “pFifoSizeInFrames” returns current number of frames in FIFO. Parameter “pNewestFrameNumber” returns number of newest frame in FIFO. Parameter “pFrameTime” returns system time (GMT) as FILETIME for the newest frame in FIFO.

## IFDCaptureLng interface

- The interface allows to obtain information about audio languages for the FDExt object. Language is an independent audio stream. You can change languages in FDConfig2 application. You can query this interface from any capturing FDExt object.

ATTENTION: Any capturing FDExt object starts capturing process on creation. It loads system resources (CPU, free system memory and PCI bus bandwidth). It is better to use interface IFDExtMgrLng (see below) to obtain language information only.

interface IFDCaptureLng : IUnknown {

HRESULT GetLngSoundFormat([out] CFDCaptureLngFormat* pFormat);

HRESULT GetLngsCount([out] long* pLngsCount);

HRESULT GetLngName([in] long Index, [out] BSTR* pName);

};

HRESULT GetLngSoundFormat([out] CFDCaptureLngFormat* pFormat);

Returns language format:

typedef enum {

FDCaptureLngFormat_Mono = 0,

FDCaptureLngFormat_Stereo = 1,

} CFDCaptureLngFormat;

Only “FDCaptureLngFormat_Stereo” is currently supported.

HRESULT GetLngsCount([out] long* pLngsCount);

Returns number of audio languages. It is possible to have zero languages.

HRESULT GetLngName([in] long Index, [out] BSTR* pName);

Returns name of language with specified index.

## IFDExtMgrLng interface

- The interface allows to obtain information about audio languages for FDExt object with specified index. Language is an independent audio stream. You can changes languages in FDConfig2 application. You can query the interface from any FDExtMgr object. The interface doesn’t load system resources (CPU, memory or PCI bus bandwidth).

interface IFDExtMgrLng : IUnknown {

HRESULT GetLngSoundFormat([in] long ObjectIndex, [out] CFDCaptureLngFormat* pFormat);

HRESULT GetLngsCount([in] long ObjectIndex, [out] long* pLngsCount);

HRESULT GetLngName([in] long ObjectIndex, [in] long Index, [out] BSTR* pName);

};

HRESULT GetLngSoundFormat([in] long ObjectIndex, [out] CFDCaptureLngFormat* pFormat);

Returns language format for FDExt object with specified index. Now it can be only “FDCaptureLngFormat_Stereo” (value equal to “1”).

HRESULT GetLngsCount([in] long ObjectIndex, [out] long* pLngsCount);

Returns number of audio languages for FDExt object with specified index. It is possible to have zero languages.

HRESULT GetLngName([in] long ObjectIndex, [in] long Index, [out] BSTR* pName);

Returns name of language with specified language index for FDExt object with specified index.

## IFDExtMgrAVObjectsInfo interface

The interface allows to obtain audio and video information for FDExt object with specified index without FDExt capture object creation. You can query the interface from FDExtMgr object.

coclass: CLSID_FDExtMgr

interface IFDExtMgrAVObjectsInfo : IUnknown {

HRESULT get_ObjectVideoParams([in] long ObjectIndex, [out] CBoardVideoParams* pVideoParams);

HRESULT get_ObjectSoundParams([in] long ObjectIndex, [out] CBoardSoundParams* pSoundParams);

HRESULT get_ObjectFrameRate([in] long ObjectIndex, [out] double* pFrameRate);

};

HRESULT get_ObjectVideoParams([in] long ObjectIndex, [out] CBoardVideoParams* pVideoParams);

Returns video parameters of capture object with specified index. Description of the structure “CBoardVideoParams” see above.

HRESULT get_ObjectSoundParams([in] long ObjectIndex, [out] CBoardSoundParams* pSoundParams);

Returns audio parameters of capture object with specified index. Description of the structure “CBoardSoundParams” see above.

HRESULT get_ObjectFrameRate([in] long ObjectIndex, [out] double* pFrameRate);

Returns video frame rate for capture object with specified index.

## Usage

The sample code below describes how to use SDK for the following tasks:

- Enumerate all valid capture devices for all FDExt boards in the system.

- Save unique identifier for selected capture object in the registry.

- Restore unique identifier from the registry and open capture object.

- Obtain video/audio parameters for the capture object.

Typically you have to add this code into your application if the application has to capture video/audio from any FDExt board.

It is recommended to create FDExtMgr object once (for example, in your C++ class constructor). The object will be used later in several places:

CComPtr<IFDExtMgr> m_pMgr;

m_pMgr.CoCreateInstance(CLSID_FDExtMgr);

Also you should create list of special classes to store information about all FDExt objects. Sample class CFDExt stores information about one FDExt capture object. It inherits the structure CFDObjectInfo.

class CFDObjInfo : public CFDObjectInfo

{

public:

CFDObjInfo(void) { m_ObjectIndex = -1; };

~CFDObjInfo(void) {};

BOOL CheckType(long nIndex,IFDExtMgr* pMgr,ULONG nLocale) {

if(m_Valid==0) return FALSE;

if(m_FDObjType == FDObjType_VideoSrc || m_FDObjType == FDObjType_VideoMixSrc)

{

m_strBoardName = CString((WCHAR*)m_BoardName);

if(pMgr) {

CComBSTR bstr;

pMgr->GetObjectFullName(this,&bstr);

m_strDevName = CString(bstr);

pMgr->TranslateFDObjectInfo(nLocale,this);

}

m_strName = CString((WCHAR*)m_BoardName) + _T(" ") +

CString((WCHAR*)m_FDObjFriendlyName);

m_ObjectIndex = nIndex;

return TRUE;

}

return FALSE;

}

long m_ObjectIndex;

CString m_strName, // full localized name for displaying to user

m_strDevName, // unique name for storing in the program settings

m_strBoardName; // board name (not localized)

};

CArray<CFDObjInfo> m_lstInfo; // list of CFDExt objects

- Enumerating all valid video capture devices. The valid device allows to capture video.

Use type “FDObjType_VideoSrc” to enumerate video capture devices for board inputs. Use type “FDObjType_VideoMixSrc” to enumerate video capture devices for board outputs.

Method “CheckType” of class CFDObjectInfo (see above) checks device type and fills additional fields.

Function “FillObjInfo” enumerates all objects in the system and creates list of all valid capture devices:

void CFDExt::FillObjInfo(ULONG nLocale)

{

if(!m_pMgr) return;

long i,n = 0;

HRESULT hr = m_pMgr->get_FDObjectsCount(&n);

if(hr!=S_OK || n<1) return;

for(i=0; i<n; i++) {

CFDObjInfo info;

hr = m_pMgr->get_FDObjectInfoByIndex(i,&info);

if(hr==S_OK && info.CheckType(i,m_pMgr,nLocale))

m_lstInfo.Add(info);

}

}

Parameter “nLocale” specifies locale identifier (LCID) for user interface language. Currently only Russian localization is supported (nLocale = 0x0419). If any other value of nLocale is specified (e.g. zero) English localization will be applied.

The method fills unique capture object name in “m_strDevName” string.

- Saving/restoring selected capture device in program settings. To display user-friendly name of capture device you need to fill your combo box with values from the field “m_strName”. To store unique name of user selected capture device you need to save field “m_strDevName” in the registry (program settings). On restoring settings you will need to read saved name from the registry and try to find the same name in the list of CFDObjectInfo structures. Then you will need to create capture object with the same unique name. If there is no object with the same unique name then you should select any other object (for example, first one in the list) and/or show warning message.

Sample code for saving/restoring selected capture device:

void CFDExt::SaveSettings() {

CFDObjInfo& info = m_lstInfo.GetAt(m_nSelDevice); // index of selected device

theApp.WriteProfileString(_T(“Settings”),_T(“SelDevice”),info.m_strDevName);

}

void CFDExt::LoadSettings() {

// function FillObjInfo() must be called before !!!

m_nSelDevice = 0; // by default, select first device in the list

CString str = theApp.GetProfileString(_T(“Settings”),_T(“SelDevice”));

for(int i=0; i<(int)m_lstInfo.GetCount(); i++) {

CFDObjInfo& info = m_lstInfo.GetAt(i);

if(info.m_strDevName==str) {

m_nSelDevice = i;

break;

}

}

}

Variable “m_nSelDevice” can be used to select corresponding device in device list (for example, in a combo box).

- Obtaining video/audio parameters of FDExt board

ATTENTION: it is recommended to obtain video/audio parameters from capture object directly (not from object with type “FDObjType_Board”). Different inputs and outputs of the same board can have different video formats. See next sample code (#4) to obtain information for specified capture object.

You have to create object with interface “IFDBoard” to obtain the board video/audio parameters. Use field “m_strBoardName” to find device with type “IFDBoard” and required board name.

Sample code:

void CFDExt::GetBoardInfo(int nDevice, // index of required capture device

long& width,long& height,double& framerate, // video format

long& nSampleRate, long& nStereo) // audio format

{

CFDObjInfo& info = m_lstInfo.GetAt(nDevice);

CComBSTR bstr(info.m_strBoardName);

CComPtr<IUnknown> pUnk; CComPtr<IFDBoard> pBoard;

if(S_OK == m_pMgr->get_FDObjectByName(bstr,FDObjType_Board,&pUnk) &&

S_OK == pUnk.QueryInterface(&pBoard) )

{

CBoardVideoParams vp;

if(S_OK == pBoard->get_BoardVideoParams(&vp))

{

width = vp.m_DX;

height = vp.m_DY;

framerate = vp.m_FrameRate;

}

CBoardSoundParams sp;

if(S_OK == pBoard->get_BoardSoundParams(&sp))

{

nSampleRate = m_SampleRate;

nStereo = sp.m_Channels/2;

}

}

}

- Use interface “IFDExtMgrAVObjectsInfo” to obtain video/audio parameters for the capture device directly. You must know index of FDExt object for required capture device. You can use field “m_ObjectIndex” (to fill device list see sample #1).

Sample code:

void CFDExt::GetObjectAVInfo(int nDevice, // index of required capture device

long& width,long& height,double& framerate, // video format

long& nSampleRate, long& nStereo) // audio format

{

CFDObjInfo& info = m_lstInfo.GetAt(nDevice);

CComQIPtr<IFDExtMgrAVObjectsInfo> pInfo(m_pMgr);

if(!pInfo) return GetBoardInfo(nDevice,width,height,framerate,nSampleRate,nStereo);

CBoardVideoParams vp;

if(S_OK == pInfo->get_ObjectVideoParams(info.m_ObjectIndex, &vp))

{

width = vp.m_DX;

height = vp.m_DY;

framerate = vp.m_FrameRate;

}

CBoardSoundParams sp;

if(S_OK == pInfo->get_BoardSoundParams(info.m_ObjectIndex, &sp))

{

nSampleRate = m_SampleRate;

nStereo = sp.m_Channels/2;

}

}