FDTM SDK – Table of Contents

FDTM SDK – description	2

How to create projects with the use of FDTM SDK	3

FDTM SDK – objects	4

FDTM SDK – Property Pages	8

FDTM SDK – Interfaces (Part 1 – FDTM board control, video input)	11

+ ISimpleGrabber	11

+ ISLTMPropertyLink	13

IPropertyLinkCallback	14

IPropertyUpdate	15

ITMSLContextSaveRestore	15

ITMSLPropertyBagSettings	16

ITMSLResources	16

ITMSLResourceHolder	17

ITMSLResourceCallBack	17

+ ISLTMSrv	17

+ ISLTMIOSettings	18

ISLTMArtist	21

ISLTMAutoKey	21

ISLTMKeyer	21

– ISLTMDVMboard	22

– ISLTMTesting	23

x ISLTMGraber	23

x ISLTMMixer	25

x ISLTMBackground	26

ISLTMSound	26

Enclosure	29

1. Typical working modes of the FDTM board’s video mixer	29

2. Video Stream Input	29

3. Standard Selection (image size and frame rate)	32

4. Showing Property Pages	33

5. Saving/restoring state of FDTM board	33

6. Format of video data in FIFO	35

7. Four-input videomixer and video playback	37

8. Example: playback of AVI file using Timeline	38

# FDTM SDK – description

FDTM SDK is intended for development of applications that use FDTM (FD300) boards for video input/output, video mixing, color-keying, controlling board’s audio mixer etc.

Important! Drivers of appropriate version for FDTM board must be installed before using of FDTM SDK, corresponding to version of the SDK itself (for example, if the folder is named FDTM_SDK_149, drivers of version 1.49 from VSDrivers_149 folder must be installed, as well as all available updates for this version).

FDTM SDK consists of the following parts:

| Part name | Where the part is located |
| --- | --- |
| Executable modules and services | Are installed in the system as the part of drivers and applications for FDTM board |
| Include header and С-files with definition of constants and objects/interfaces identifiers | Folders [SLTMSrvd], [SLTMKeyServer], [SLTMVideoRenderer] Must be included in project |
| Documentation – description of FDTM board, objects and interfaces | Folder [Docs], and files Readme_Rus.txt and Readme_Eng.txt |

Detailed description of FDTM (FD300) board, FDTM SDK objects and interfaces can be found in the following documents:

| Document | Contains |
| --- | --- |
| FDTM_SDK_BoardDescription | Description and schemes of modules of FDTM (FD300) board |
| SLTMVideoRenderer | Description of SLTM Video Renderer |

Description of the main objets and interfaces of FDTM SDK objects and interfaces can be found in this document.

# How to create projects with the use of FDTM SDK

Program modules of FDTM SDK are COM-objects that are installed and registered in the system as a part of drivers and applications of FDTM board. The COM technology is used for calling FDTM objects, so application must call CoInitializeEx(NULL, COINIT_MULTITHREADED) in the beginning of each working thread, otherwise any posterior call (and even creation of instance of an object) of any interface function will return error «CoInitialize must be called first». Accordingly, CoUnitialize() must be called on the finish of every working thread.

It is recommended to create projects using Microsoft Visual Studio version 6.0 or higher, with the use of ATL library. COM initialization will be inserted to an ATL project automatically. Besides, ATL library provides a set of functions and classes that significantly facilitate development of COM-projects (for example, CComPtr class – “smart pointer” that automatically counts object reference counter).

If you create an ATL project, support of ATL library is inserted automatically to it. However, there may be a need in creation of an MFC Project, that does not contain support of ATL by default. In order to insert support of ATL to an MFC project, press right mouse button over classes tree of the project, and select menu item “New ATL Object”. A message will appear: “Do you want to add ATL to your MFC project?”. Answer “Yes” and press “Cancel” in a next dialog (if it appears). The files needed for support of ATL will be automatically inserted to the project.

In order to use objects of FDTM SDK in an ATL project, it is needed to:

- Copy entire folder [SLTMSrvd] with all include h- и С-files to a folder with project.

- Include “sltmsrvd.h” header file in the main header file of the project, and include file “sltmsrvd_i.c” in the main implementation file of the project (#include “sltmsrvd.h”, #include “sltmsrvd_i.c”). The rest files must not be included, but all them must be located in the same folder.

- You may include IDL-files from [IDL] folder with interface definitions to the project to see names of interfaces, methods and properties in the class view of Visual Studio. Don’t forget to turn off the flag «Exclude file from Build» for all IDL-files, because they must not be compiled. To do this, add IDL-files in the “File View” of the project from [IDL] folder, select all added IDL-files, press right mouse button, select “Properties”, turn off the flag in the properties dialog and press “Ok”.

Remarks:

The folder [SLTMSrvd] contains files with definitions of the main objects and interfaces. It should be enough in the majority of all cases. However, there are two additional folders: [SLTMKeyServer] – include files for use of color-keying server object, and [SLTMVideoRenderer] – include files for DirectShow video playback applications that use SLTM Video Renderer. In case if you need to create projects that use color-keying server object or SLTM Video Renderer, copy corresponding folder [SLTMKeyServer] or [SLTMVideoRenderer] and include corresponding h- and C-files to the project.

# FDTM SDK – objects

coclass SLTMSimpleGrabber

{

interface ISimpleGrabber;

};

coclass SLTMVS2000Grabber

{

interface ISimpleGrabber;

interface IVS2000GrabberConfigure;

};

The SLTMSimpleGrabber and SLTMVS2000Grabber objects implement the ISimpleGrabber interface, which is intended for simple capture of video data to PC. The SLTMSimpleGrabber object aggregates the SLTMSrv and TMSLPropertyBag objects used for configuring the FDTM board’s parameters.

The ISimpleGrabber interface provides complete functionality needed for video capture in a standard application. It allows to enumerate all FDTM boards installed in the system; select and acquire a desired board for exclusive video input; start video streaming (capturing) from input or output of internal mixer of the FDTM board. Video data are buffered in PC’s memory by fields. The interface allows to get information about current number of fields in the buffer, detailed information about each field (size, parity, count number), and to copy (convert) one field or frame to user’s buffer in required format (UYVY, RGBA or RGB). Note that you must call a special function to indicate that you have finished work (copying, conversion etc.) with the current frame for the buffer to be emptied, and the FDTM board could continue data input to emptied space in the buffer.

The IVS2000GrabberConfigure interface implemented by the SLTMVS2000Grabber object is intended for deinterlacing input video data. It allows to «increase» vertical image resolution by reconstructing full progressive-scan frame from the current field. This object is intended for using with virtual studio only.

coclass TMSLPropertyBag

{

interface ITMSLPropertyBagSettings;

interface IPropertyBag;

};

The TMSLPropertyBag object implements the IPropertyBag interface over the system registry, and path in the registry can be set via the ITMSLPropertyBagSettings interface. The object allows to save/restore settings of the FDTM board to/from the registry (see the description of the ITMSLContextSaveRestore interface of the SLTMSrv object). Example of source code can be found in Enclosure “5. Saving/restoring state of FDTM board”.

coclass SLTMAutoKey

{

interface ISLTMAutoKey;

};

The SLTMAutoKey object is intended for automatic tuning the color-keying parameters. Lighting and background must be set for its correct working, i.e. the selected video-input of the FDTM board must be connected to a camera, pointed to a blue or green uniformly-lighted background.

coclass SLTMSrv

{

interface ISLTMSrv;

interface ISLTMIOSettings;

interface ISLTMGraber;

interface ISLTMKeyer;

interface ISLTMMixer;

interface ISLTMArtist;

interface ISLTMBackground;

interface ISLTMTesting;

interface ISLTMDVMboard;

interface ITMSLResources;

interface ITMSLLockedMemoryManager;

interface ITMSLContextSaveRestore;

interface ISLTMSound;

};

SLTMSrv is a basic object that must be created for working with FDTM board, and it implements all basic interfaces. The most commonly used interfaces are: ITMSLResources, ISLTMMixer, ISLTMArtist, ISLTMKeyer, and possibly ITMSLContextSaveRestore.

ISLTMSrv allows to select a desired board by index (starting from 0). If you want to change the boards order in the system, change the following keys of the DWORD type in the registry with the path «HKEY_LOCAL_MACHINE\SOFTWARE\Darim Vision\SLTM\Params» to new indices of the boards, for example:

Dsp_Ind_0 = 2

Dsp_Ind_1 = 0

Dsp_Ind_2 = 1

In this case the first board (index 0) will become last one (index 2), and the order of the other boards will be shifted forward.

The ISLTMIOSettings interface is intended for setting parameters of inputs and outputs of the FDTM board, for example video-standard PAL/NTSC, Hue, Saturation, Contrast etc., selecting types of inputs and output (CVBS/S-Video/YUV/RGB/), genlock settings, etc.

The ISLTMGraber interface is intended for capturing video data from FDTM to PC memory. This interface allows to create FIFO for data input and start video capturing from input or output of internal mixer of the board. Video data are stored in FIFO in a «raw» format by fields, depending on the settings of the current input or output of the internal mixer. The field data can get over the end of FIFO and be continued at the start of FIFO, dividing into two parts. The application in DSP TriMedia skips (does not write to FIFO) regular field if there is no free space in FIFO for it. So capture application if PC must «pull out» data from FIFO in order to not to lose input data. This interface is a base for implementation of the SLTMSimpleGrabber object. However, we recommend to use the ISimpleGrabber interface of the SLTMSimpleGrabber object as more simple in use.

The ISLTMMixer interface controls the modes of the internal mixer of the FDTM board that performs mixing of video data from video input(s) and scene (video-decoration from PC).

The ISLTMBackground interface controls passing of video-decoration from PC to the FDTM board – «scene» for the internal mixer. Currently only one FIFO can be created for video playback, and video data in it must be in a «raw» format (one of SLTM_F16YUV, SLTM_F16YAUV, SLTM_F16YUVYUV or SLTM_F16YAUVYUV).

The ISLTMArtist interface is intended for selecting a type of input data: only one input A or B, only one input A or B with alpha-channel, both A and B inputs, one input with alpha-channel and second input without alpha-channel.

The ISLTMKeyer interface is intended for loading keying table, switching of active table, and setting additional keying parameters. Currently the setting of vertical mask filtration mode is not implemented in it.

The ITMSLResources interface is intended for locking the resources of the FDTM board, in order to separate access to resources between different parts of the board between various applications. While locking a resource, the application must give reference to its TMSLResourceHolder object (it must be created before). It is required that the driver of the FDTM board could check whether resource has been already locked (and locker application still needs this resource) or not, or possibly locker application has been already finished. Currently, the resources are divided into rc_artist, rc_scene, rc_mixer, rc_output, and rc_graber.

The ISLTMSound interface allows to control the audio mixing parameters of the FDTM board: change volume levels of inputs, outputs and cross-mixing of inputs to outputs, set delay of sound on inputs, etc.

The ISLTMDVMboard interface is intended for controlling of the DVM board, connected to the FDTM board via a large 40-pin connector.

The ITMSLContextSaveRestore interface allows to save/restore settings of the FDTM board via the standard IPropertyBag object. Use TMSLPropertyBag for saving/restoring settings to/from the registry.

The ITMSLLockedMemoryManager is a special service interface and is intended for allocating memory block accessible from PC and from DSP TriMedia.

ISLTMTesting provides output of different service information and is intended for working with devices on the I2C bus. It is not used in common applications.

coclass TMSLResourceHolder

{

interface ITMSLResourceCallBack;

interface ITMSLResourceHolder;

};

The TMSLResourceHolder object is created for locking resources of the FDTM board. When an object tries to lock some resources of the board, the driver calls the ITMSLResourceCallBack interface of an object that has locked the same resource before, and asks to unlock the resource.

coclass TMSLDriver

{

interface ITMSLDriver;

};

The TMSLDriver object is not used in user applications and can be removed in future at all.

coclass TMSLLockedMemoryObject

{

interface ITMSLLockedMemoryObject;

};

TMSLLockedMemoryObject is a service object and is not used in user applications.

coclass SLTMMPEGPlayer

{

interface ISLTMMPEGPlayer;

};

coclass SLTMMPEGScript

{

interface ISLTMMPEGScript;

};

The SLTMMPEGPlayer and SLTMMPEGScript objects are intended for playback of MPEG-compressed video-decorations. These objects are currently not supported by the FDTM boards and will be possibly significantly re-designed in future.

# FDTM SDK – Property Pages

There are several PropertyPages in the FDTM objects intended for setting the FDTM boards parameters. All of them are of the same size and can be used simultaneously (see also Enclosure “4. Showing Property Pages”).

LinePropertyPage allows to select video input (CVBS1/2/…, S-Video1/2/…, YUV1/2, etc.), change Hue, Saturation, and other parameters for each video input.

FormatPropertyPage allows to select the current FDTM board by number, video standard PAL/NTSC, and other parameters corresponding to both inputs and output of the FDTM board.

OutputPropertyPage allows to select type of output (CVBS, S-Video, RGB, and others), genlock settings, etc.

DVMPropertyPage allows to change parameters of the DVM board, connected to FDTM board via large 40-pin connector.

SoundPropertyPage allows to control sound settings of the FDTM board: change volume levels of inputs, outputs, and cross-mixing, and set audio delay for each input line. All volume controls are divided into 6 groups – 3 input groups and 3 output ones. Each group contains one input or output volume and 3 volumes of cross-mixing from inputs to outputs. Thus, each cross-mixing volume control is present in two groups one input and one output. Selecting a group, you can see all volume controls corresponding to its line.

There is a special LinePropertyLink object used for transferring parameters to any PropertyPage. Using this object you must transfer the following parameters to each PropertyPage: reference to the SLTMSrv object, number of input line for LinePropertyPage, and functionality restriction flags for the following parameters: PL_Format – prohibition to change video-format PAL/NTSC; PL_HalfSize – half resolution only; PL_BoardIndex – prohibition to change board number; PL_LineIndex – prohibition to change line number; PL_FieldHFullRes and PL_FieldHHalfRes – the way how to count size of field and frame; PL_CheckSecurity – check restrictions of the current FDTM board’s registration, and reflect them in all PropertyPages. See description of the ISLTMPropertyLink interface for more details.

The SoundMixer object, as well as SoundPropertyPage, has a PropertyPage too, and is intended for setting all audio mixing parameters, but it also has much larger size because it shows all parameters simultaneously.

The SLTMCaptureAdvSettings object has a PropertyPage for video capture object.

Examples on how to use property pages can be found in the Enclosure “4. Showing Property Pages”.

# FDTM SDK – Interfaces (Part 1 – FDTM board control, video input)

Remarks:

Interface titles can be preceded by additional signs:

+ (plus)	such interface is highly recommended to use; some corresponding tasks can’t be performed without use of this interface; it is recommended to pay attention to this interface in any case.

– (minus)	such interface is not recommended to use, because it is intended for some service needs, and improper use of it can result serious problems (right up to unxepected hardware malfunction), or this is a low-level interface, and it is much better to use some higher-level interface that is marked with «+» sign instead.

(no sign)	such interface is used only if needed.

х (cross)	such inteface is highly not recommended to use (for example, it is outdated, and some newer interface should be used instead); description of this interface is included only in case if there is a need in use of it because of some technical reasons.

The first part contains descriptions of some service low-level interfaces that allow to perform the following actions with FDTM board:

- select current board and change some its settings;

- save and restore settings of selected board;

- tune parameters of video inputs and outputs;

- control audio mixer;

- lock/unlock board resources;

- create links with property pages;

- perform color-keying;

- perform low-level video input and playback;

- perform some other service actions.

## + ISimpleGrabber

Object: CLSID_SLTMSimpleGrabber

HRESULT GetBoardCount([out] long *BoardsCount) – returns number of the FDTM boards installed in the system.

HRESULT GetBoardStatus([in] long BoardIndex, [out] SGStatus *Status, [out] long *Version) – returns state of the FDTM board with specified index (number) and its driver version number (FDTM software version). The status can be one of the following: SG_None – no board, SG_Free – board is present and is free for use, SG_Busy – board is present, but is in use by some application. DWORD Version consists of two WORDs, higher one contains a driver version number, and lower contains the FDTM software version number. Current Version is 0x01010101.

HRESULT AcquireBoard([in] long BoardIndex) – selects a specified board for further work with and locks its rc_graber resource. Returns the E_INVALIDARG error code if the board is already locked by some application. This function must be called before any call of other functions. If the board was not locked with AcquireBoard, all subsequent calls of any other functions will return the E_ACCESSDENIED error code.

HRESULT FreeBoard() – frees the rc_graber resource and disconnects from a current board.

HRESULT GetBoardInfo([out] long* dx, [out] long* dy, [out] double* FrameRate) – returns a current field size (half frame) and frame rate (double field rate) for acquired board. The contents of FIFO is not emptied when PAL/NTSC format is changed (or frame width), so it is possible that size returned by this function does not correspond to real size of fields currently placed in FIFO. Use the GetFieldInfo function to get a real size of fields currently placed in FIFO.

HRESULT GetBoardFieldNumber([out] long *HardwareFieldNumber) – returns absolute time (in fields). Currently it is emulated on PC's system clock.

HRESULT SwitchChannel([in] SGSource Src) – makes active the first (if Src parameter is SG_Artist_1) or second (if Src parameter is SG_Artist_2) decoder, and switches to corresponding keying table (first or second). Note that Src must be SG_Artist_1 or SG_Artist_2 only!

HRESULT CaptureField([in] SGSource Src, [out] SGFieldPosition *FieldPos, [in] SGPixelFormat Format, [out, size_is(cb)] BYTE* VideoData, [in] long cb, [in] long stride) – grabs one field and copies it to a specified buffer. If the grab stream has been already started, the function will return the E_ACCESSDENIED error code. See also remarks 1, 2 and 3.

HRESULT CaptureFrame([in] SGSource Src, [in] SGPixelFormat Format, [out, size_is(cb)] BYTE* VideoData, [in] long cb, [in] long stride) – grabs one frame (two fields) and copies it to a specified buffer. If the grab stream has been already started, the function will return the E_ACCESSDENIED error code. See also remarks 1, 2 and 3.

HRESULT StartStream([in] SGSource Src, [in] long FifoSizeInFields, [in] long UseColorCorrection) – allocates FIFO for a specified number of fields and starts video capture (see remark 1) from the FDTM board. If the long UseColorCorrection parameter is not set to zero, the color correction process will be also activated, and the video stream being captured will contain alpha-channel. If it is set to zero, the video stream being captured will not contain alpha-channel, and color correction won’t be activated. See also remark 4.

HRESULT StopStream() – stops video capture and releases (destroys) FIFO.

HRESULT CaptureFields([in] SGSource Src, [in] long FieldsCount, [in] long UseColorCorrection) – allocates grab FIFO for a specified number of frames (double fields) and captures a specified number of fields from the FDTM board. The SGSource Src (see remark 1) and long UseColorCorrection parameters correspond to the parameters in the StartStream function. See also remark 4.

HRESULT GetStreamInfo([out] long *NumberOfCapturedFields, [out] long *NumberOfFreeFields, [out] long *FirstFieldNumber, [out] long *LastFieldNumber) – returns information about a current state of FIFO: number of captured frames in FIFO (NumberOfCapturedFields); approximate number of fields free for subsequent video data (NumberOfFreeFields); index numbers of the first and last captured fields (FirstFieldNumber and LastFieldNumber).

HRESULT SkipFields([in] long SkipCount) – frees space occupied by the first SkipCount fields in FIFO.

HRESULT GetFieldInfo([in] long FieldIndex, [out] long* FieldNumber, [out] SGSource *Src, [out] SGFieldPosition *FieldPos, [out] long* dx, [out] long* dy) – returns information about field with a specified index (starting from 0) in FIFO: field number (from system start) in FieldNumber; input line number – value SG_Artist_1 for decoder A or value SG_Artist_2 for decoder B in Src parameter; field parity (upper or lower field) in FieldPos parameter; field size (width and height) in dx and dy parameters.

HRESULT GetFieldImage([in] long FieldIndex, [in] long x0, [in] long y0, [in] long dx, [in] long dy, [in] SGPixelFormat Format, [out, size_is(cb)] BYTE* VideoData, [in] long cb, [in] long stride) – converts a specified part of an image from a specified field to the buffer. Coordinates are from the top left corner of the image. See also remark 3.

HRESULT GetFrameImage([in] long UpperFieldIndex, [in] long LowerFieldIndex, [in] long x0, [in] long y0, [in] long dx, [in] long dy, [in] SGPixelFormat Format, [out, size_is(cb)] BYTE* VideoData, [in] long cb, [in] long stride) – converts a specified part of a frame from two specified fields to the buffer. Coordinates are from the top left corner of the image. See also remark 3.

HRESULT Debug() – is intended for testing purposes. Currently does nothing.

Remarks:

- SGSource Src parameter can’t be SG_Background, only SG_Artist for video capture from video input and SG_MixOut for video capture from the board’s output to PC.

- The function returns field parity (upper or lower field) in the SGFieldPosition *FieldPos parameter.

- The image is being converted to the format specified in SGPixelFormat Format: SG_UYVY – standard format YUV4:2:2 (byte order in stream is UYVYUYVY…); SG_BGRA/SG_RGBA – 4 bytes per pixel in order common for Windows systems (byte order in stream is BGRABGRA…); SG_BGR/SG_RGB – 3 bytes per pixel in order common for Windows systems (byte order in stream is BGRBGR…). Possibly there will be separation of formats SG_BGRA and SG_RGBA in future, as well as SG_BGR and SG_RGB. The BYTE* VideoData parameter points to data buffer start. Parameter long stride defines offset (in bytes) to start of next line in output image. The image in the FDTM board comes from top to bottom, whereas in Windows Bitmap – from bottom to top, so you can set negative value of parameter long stride. The function will mirror the specified part of the image vertically in this case, and the last line in buffer BYTE* VideoData will contain the first line (the topmost one) of the image (within a specified rectangle).

- In order to start of video capture will be possible, there must be a registration that enables video capture loaded in the FDTM board. If there is no such registration, the StartStream command will be executed successfully and will return S_OK, but there will be no data being accumulated in FIFO (and the GetStreamInfo function will always return zero value in the NumberOfCapturedFields parameter). The CaptureFields command provides capture of a specified number of fields (but no more than 10) without registration. It can help when there is no need to enable video stream capture, but it is required to capture some test frames (for example, for tuning color-keying parameters).

## + ISimpleGrabberData

Object: CLSID_SLTMSimpleGrabber

This interface allow to capture two video streams, audio & VBI data. To select input types uses type SGFieldDataType (see remark 1).

HRESULT StartStreamAdv([in] SGSource Src, [in] long FifoSizeInFields, [in] long UseColorCorrection, [in] long enable_data_bits) – is the same as method StartStream of interface ISimpleGrabber, but it has additional parameter enable_data_bits, which specifies required data types (see remark 1).

HRESULT GetFieldDataInfo([in] long FieldIndex, [out] SGFieldDataType* data_bits) – return all available data types for the field with local fifo index FieldIndex.

HRESULT GetVBIData([in] long FieldIndex, [in] SGFieldDataType vbi_type, [out, size_is(cb)] BYTE* VBIData, [in] long cb) – copy one line of raw VBI data of specified type vbi_type from the field with specified index into specified buffer. VBI data is Y-data only (one byte per pixel). Width of the VBI line is the same as image width (720 pixels as usual).

HRESULT GetAudioDataInfo([in] long FieldIndex, [out] long* sample_count, [out] long* sample_rate, [out] long* channel_count, [out] long* complete_sample_size) – return format and number of audio data for specified field.

sample_count – number of audio samples.

sample_rate – sample rate.

channel_count – audio channel number.

complete_sample_size – complete size (in bytes) of one sample (for all channels).

You have to read sample_count* complete_sample_size bytes of audio data.

HRESULT GetAudioData([in] long FieldIndex, [out, size_is(cb)] BYTE* AudioData, [in] long cb) – copy audio data into specified buffer. The buffer will includes audio data for all channels (6 mono channels as usual).

HRESULT GetFieldImageAdv([in] long FieldIndex, [in] long x0, [in] long y0, [in] long dx, [in] long dy, [in] SGPixelFormat Format, [out, size_is(cb)] BYTE* VideoData, [in] long cb, [in] long stride, [in, defaultvalue(SG_Video1)] SGFieldDataType imageType) – is the same as method GetFieldImage of interface ISimpleGrabber convert part of source image into specified buffer. Last parameter imageType specifies input video type (first or second video stream) (see remark 1).

HRESULT GetFrameImageAdv([in] long UpperFieldIndex, [in] long LowerFieldIndex, [in] long x0, [in] long y0, [in] long dx, [in] long dy, [in] SGPixelFormat Format, [out, size_is(cb)] BYTE* VideoData, [in] long cb, [in] long stride, [in, defaultvalue(SG_Video1)] SGFieldDataType imageType) – is the same as method GetFrameImage of interface ISimpleGrabber, except last parameter specifies input video type (first or second video stream) (see remark 1).

HRESULT GetFieldImageAsDIB([in] long FieldIndex, [in] long x0, [in] long y0, [in, size_is(cbBmpInfo)] BYTE *pBmpInfo, [in] long cbBmpInfo, [out, size_is(cbBmpData)] BYTE* pBmpData, [in] long cbBmpData, [in, defaultvalue(SG_Video1)] SGFieldDataType imageType) – convert specified part of source image info standard bitmap data:

pBmpInfo – pointer to structure BITMAPINFOHEADER.

cbBmpInfo – size of structure BITMAPINFOHEADER.

pBmpData – pinter to image buffer.

cbBmpData – size of image buffer.

HRESULT GetFrameImageAsDIB([in] long UpperFieldIndex, [in] long LowerFieldIndex, [in] long x0, [in] long y0, [in, size_is(cbBmpInfo)] BYTE *pBmpInfo, [in] long cbBmpInfo, [out, size_is(cbBmpData)] BYTE* pBmpData, [in] long cbBmpData, [in, defaultvalue(SG_Video1)] SGFieldDataType imageType) – is the same as method GetFieldImageAsDIB, except it convert two source fields into one frame buffer.

HRESULT GetFieldImageAs420Planar([in] long FieldIndex, [in] long x0, [in] long y0, [in] long dx, [in] long dy, [out, size_is(cbY)] BYTE* DataY, [in] long cbY, [in] long strideY, [out, size_is(cbU)] BYTE* DataU, [in] long cbU, [in] long strideU, [out, size_is(cbV)] BYTE* DataV, [in] long cbV, [in] long strideV, [in, defaultvalue(SG_Video1)] SGFieldDataType imageType) – convert source image into three separate buffers (Y, U, V). Y buffer is full size buffer – width and height of the buffer is the same as image width & height. U & V buffers has half height & half width.

HRESULT GetFrameImageAs420Planar([in] long UpperFieldIndex, [in] long LowerFieldIndex, [in] long x0, [in] long y0, [in] long dx, [in] long dy, [out, size_is(cbY)] BYTE* DataY, [in] long cbY, [in] long strideY, [out, size_is(cbU)] BYTE* DataU, [in] long cbU, [in] long strideU, [out, size_is(cbV)] BYTE* DataV, [in] long cbV, [in] long strideV, [in, defaultvalue(SG_Video1)] SGFieldDataType imageType) – is the same GetFieldImageAs420Planar, except it convert two source fields into one frame buffer.

HRESULT GetFieldImageAs420Planar2([in] long FieldIndex, [in] long x0, [in] long y0, [in] long dx, [in] long dy, [out, size_is(cbY)] BYTE* DataY, [in] long cbY, [in] long strideY, [out, size_is(cbU)] BYTE* DataU, [in] long cbU, [in] long strideU, [out, size_is(cbV)] BYTE* DataV, [in] long cbV, [in] long strideV, [in, defaultvalue(SG_Video1)] SGFieldDataType imageType) – is the same GetFieldImageAs420Planar, except it integrate U & V data vertically into the field only (across the line, not the nearest lines).

HRESULT GetFrameImageAs420Planar2([in] long UpperFieldIndex, [in] long LowerFieldIndex, [in] long x0, [in] long y0, [in] long dx, [in] long dy, [out, size_is(cbY)] BYTE* DataY, [in] long cbY, [in] long strideY, [out, size_is(cbU)] BYTE* DataU, [in] long cbU, [in] long strideU, [out, size_is(cbV)] BYTE* DataV, [in] long cbV, [in] long strideV, [in, defaultvalue(SG_Video1)] SGFieldDataType imageType) – is the same GetFieldImageAs420Planar2, except it convert two source fields into one frame buffer.

Remarks:

- 1. Type SGFieldDataType:

SG_Video1,	video data from active line (for example LineA for mode LineA+LineB)

SG_Video2,	video data from inactive line (for example LineB for mode LineA+LineB)

SG_Video4Artist_1,	video data from line LineA only

SG_Video4Artist_2,	video data from line LineB only

SG_Alpha,	alpha channel (one byte per pixel)

SG_Audio,	audio data

SG_VBI_LINE_1..SG_VBI_LINE_8,	VBI data (lines under visible area)

## + ISLTMPropertyLink

Object: CLSID_SLTMPropertyLink

HRESULT GetBoardCount([out] long *BoardsCount) – returns number of the FDTM boards installed in the system.

Property IUnknown* Server – gets/puts reference to object SLTMSrv for all Property Pages.

Property long Line – gets/puts number of a current input line (0 or 1) for LinePropertyPage.

Property long Changed – gets/puts a flag of change of any board's parameter.

Property long Restriction – gets/puts restrictions for the board parameters’ change and other settings that affect the corresponding PropertyPages. The following restriction flags can be used:

Restriction flags for FormatPropertyPage :

PL_Format 	prohibited to change a current video format of the FDTM board (PAL/NTSC) – and current vertical resolution too.

PL_HalfSize 	prohibited to change a current horizontal resolution.

PL_BoardIndex 	prohibited to change a current board number.

PL_FieldHFullRes	shows a field height instead of a frame height for full horizontal resolution – for example, 720х288 instead of 720х576.

PL_FieldHHalfRes 	shows a field height instead of a frame height for a half horizontal resolution – for example, 352х288 instead of 352х576.

The restriction flags for LinePropertyPage :

PL_LineIndex 	prohibited to change a current line (channel) number.

The restriction flags for Property Pages :

PL_CheckSecurity 	checks prohibited modes of the board and enables/disables the corresponding parameters – for example, in the list of inputs only the enabled ones will be shown, and YUVn and RGBn will be excluded if the registration does not allow to work with YUV inputs.

The IUnknown* Callback property – gets/puts reference to the IPropertyLinkCallback interface that allows you to get callback reaction if the user has changed any parameters in Property Pages.

The IUnknown* Update([in] long pageID) property – gets/puts reference to the IPropertyUpdate interface (that is present in all Property Pages) for a specified property page (see enclosure 1). This function returns non-zero pointer if the corresponding page has been initialized before.

Remarks:

- 1. Types of the Property Pages:

PL_LinePage,	LinePropertyPage

PL_FormatPage,	FormatPropertyPage

PL_OutputPage,	OutputPropertyPage

PL_DVMPage,	DVMPropertyPage

PL_SoundPage	SoundPropertyPage

## IPropertyLinkCallback

HRESULT PropertyPageChanging([in] long page_id, [in] long param_id) – is called before changing the «param_id» parameter in PropertyPage «page_id».

HRESULT PropertyPageChanged([in] long page_id, [in] long param_id) – is called after changing the «param_id» parameter in PropertyPage «page_id».

List of possible «page_id»:

typedef enum {

PL_LinePage,	LinePropertyPage

PL_FormatPage,	FormatPropertyPage

PL_OutputPage,	OutputPropertyPage

PL_DVMPage,	DVMPropertyPage

PL_SoundPage	SoundPropertyPage

} PropertyLinkPageID;

List of possible «param_id»:

typedef enum {

PL_LinePage_Line=1000,		line (channel) number

PL_LinePage_Input,			input (CVBS1, S-Video3, YUV5, …)

PL_LinePage_InputParams,		parameters of input (Hue, Saturation, …)

PL_LinePage_All=1999,		some (or all) parameters of LinePropertyPage

PL_FormatPage_BoardIndex=2000,	board index

PL_FormatPage_Standart,		video format (PAL/NTSC)

PL_FormatPage_Resolution,		horizontal resolution

PL_FormatPage_All=2999,		some (or all) parameters of FormatPropertyPage

PL_OutputPage_Type=3000,		output (CVBS, S-Video, YUV, …)

PL_OutputPage_Bus,			output bus (input of output video-coder)

PL_OutputPage_Genlock,		genlock source (master, line 1 or line 2)

PL_OutputPage_GenlockParams,	genlock parameters (X and Y offset)

PL_OutputPage_All=3999,		some (or all) parameters of OutputPropertyPage

PL_DVMPage_All=4999,		some (or all) parameters of DVMPropertyPage

PL_SoundPage_InputDelay=5000,	delay of input audio lines

PL_SoundPage_InputVolume,		hardware input sound volume

PL_SoundPage_MixVolume,		mix sound volume of input line to output

PL_SoundPage_OutputVolume,		hardware output sound volume

PL_SoundPage_All=5999		some (or all) parameters of SoundPropertyPage

} PropertyLinkParamID;

## IPropertyUpdate

HRESULT Update([in] long param_id, [in] long save_new_param) – this method is called after changes of some board's parameters to reflect these changes in Property Page. The second parameter indicates that new parameters must be used as defaults, and the Cancel button should restore their values instead of previous ones. The param_id parameter corresponds to one of the methods of the IpropertyLinkCallback interface.

HRESULT BoardChanged() – reports to the property page that index of a current board has been changed, and all parameters should be re-read anew.

HRESULT RestrictionChanged() – reports to the property page that the restrictions specified in the LinePropertyLink object have been changed, and the restrictions should be re-read anew.

## ITMSLContextSaveRestore

Object: CLSID_SLTMSrv

HRESULT Load([in] IPropertyBag *pPropBag) – restores a state of the FDTM board from a specified container (all parameters property pages LinePropertyPage, FormatPropertyPage, and OutputPropertyPage).

HRESULT Save([in] IPropertyBag *pPropBag) – saves a state of the FDTM board to a specified container.

HRESULT LoadPropertyPageInfo([in] long pageID, [in] IPropertyBag *pPropBag) – restores a specified part of the FDTM variables, defined by the given PropertyPage identifier (see remark 1), from a specified container.

HRESULT SavePropertyPageInfo([in] long pageID, [in] IPropertyBag *pPropBag) – saves a speсified part of the FDTM variables, defined by the given PropertyPage identifier (see remark 1), to a specified container.

Remarks:

- 1. Types of Property Pages:

PL_LinePage,	LinePropertyPage

PL_FormatPage,	FormatPropertyPage

PL_OutputPage,	OutputPropertyPage

PL_DVMPage,	DVMPropertyPage

PL_SoundPage	SoundPropertyPage

2. Refer to Enclosure “5. Saving/restoring state of FDTM board” to see the source code for saving/restoring of a state of all FDTM boards.

## ITMSLPropertyBagSettings

Object: CLSID_TMSLPropertyBag

HRESULT SetRegistryKey([in] DWORD hKey, [in] BSTR KeyStr) – sets path in the registry for data saving/restoring using the IpropertyBag interface. For example, hKey can be HKEY_LOCAL_MACHINE, and KeyStr contains sub-key path (for example, "SOFTWARE\\Darim Vision\\SLTM\\BoardSettings").

## ITMSLResources

Object: CLSID_SLTMSrv

HRESULT LockResources([in] long mask, [out] long* locked_mask, [in] IUnknown* CallBack) – tries to lock resources specified by the mask (see remark 1). Returns the mask of successfully locked resources. Pointer to non-null CallBack object must be present (see remark 2).

HRESULT UnlockResources([in] long mask, [in] IUnknown* CallBack) – frees resources specified by the mask (see remark 1). Pointer to non-null CallBack object must be present (see remark 2).

HRESULT GetLockedResources([out] long* mask, [in] IUnknown* CallBack) – returns the mask of currently locked resources. Pointer to non-null CallBack object must be present (see remark 2).

HRESULT CheckUnlockResources() – checks whether currently locked resources are still locked by some objects using the references to the CallBack objects given by them (see remark 2). Also asks objects to unlock the resources locked by them (but it is not guaranteed that resources will be really unlocked).

HRESULT GetLockedResources_2([out] long* mask_owner, [out] long* mask_other, [out] long* mask_free, [in] IUnknown* CallBack) – returns a detailed information about locked resources: mask_owner – mask of resources, locked by owner of the given CallBack object; mask_other – mask of resources, locked by other objects (not owners of the given Callback object); mask_free – mask of resources that are currently free.

Remarks:

- 1. The types of the FDTM board resources:

rc_artist	controls input video data (types and settings of inputs)

rc_scene	playbacks FIFO (controls video data output from PC memory to the FDTM board)

rc_mixer	controls the video mixer

rc_output	controls the output video coder and output digital bus

rc_graber	captures FIFO (controls video data input from the FDTM board to PC memory)

- 2. For more information about the use of the CallBack object for resource locking/unlocking refer to the description of the ITMSLResourceCallBack and ITMSLResourceHolder interfaces.

## ITMSLResourceHolder

Object: CLSID_TMSLResourceHolder

HRESULT SetCallBack([in] IUnknown* pIUnk) – sets pointer to the ITMSLResourceCallBack interface that is used for unlocking locked resources (see remark 1).

## ITMSLResourceCallBack

Object: CLSID_TMSLResourceHolder

HRESULT TryUnlockResources([in] long mask) – the server asks to unlock a specified resource.

HRESULT TestLockeResources() – the server checks whether the object is «alive». The object must return S_OK to confirm this.

HRESULT GetGUID([out] GUID* pGUID) – the server asks for identifier (GUID) of an object to identify locker of a resource. The identifier must be unique for each instance of an object.

Important: In order to lock resources you must create the TMSLResourceHolder object, which includes the ITMSLResourceHolder and ITMSLResourceCallBack interfaces. If you want to control the situation when other object tries to lock a resource, you must implement the ITMSLResourceCallBack interface and give it to the TMSLResourceHolder object as a parameter of the SetCallBack function of the ITMSLResourceHolder interface. The TMSLResourceHolder object will call only the TryUnlockResources method of your interface. When the TMSLResourceHolder object is released (destroyed), the resources will NOT be unlocked automatically.

Remarks:

1. The description of the resource locking/unlocking process of the FDTM board:

There is a mask of the locked resources for each FDTM board in the SLTMSrvs.exe server. For each locked resource there is a pointer to interface, given as a CallBack parameter in the LockResources function of ITMSLResources interface.

When an object tried to lock an already locked resource, or when the CheckUnlockResources function of the ITMSLResources interface is called, the server will try to call the TryUnlockResources method of the ITMSLResourceCallBack interface for the resource. An object called may ignore this call or unlock the resource, but it must return S_OK in any case. If the call of TryUnlockResources or TestLockeResources returns an error code, the server counts that the object being called is «dead», and unlocks the resource.

The GetGUID method of the ITMSLResourceCallBack interface must return a unique identifier for each instance of the object (you can create GUID with the UuidCreate function – see MSDN for details), as this identifier is used by the server to check that the same object (or not the same) tries to lock an already locked resource.

## + ISLTMSrv

Object: CLSID_SLTMSrv

HRESULT get_BoardsCount([out] long* pVal) – returns number of the FDTM boards installed in the system.

HRESULT set_SelectedBoardIndex([in] long BoardIndex) – selects a board with a specified index to further work.

HRESULT get_SelectedBoardIndex([out] long* BoardIndex) – returns index of a currently selected board.

HRESULT AquirBoard() – locks a current board (currently does nothing).

HRESULT FreeBoard() – frees a current board for other clients to work.

HRESULT get_Version([out] long* pVal) – returns the FDTM application version (current version is 0х01010001). The first (higher) two bytes contain application version number – high and low parts of version number. There can be significant change of board interfaces on change of high part of version number. Last word contains build number of application.

HRESULT get_Status([out] long* pVal) – returns the FDTM board's state (0 if everything is OK).

## + ISLTMIOSettings

Object: CLSID_SLTMSrv

HRESULT set_VideoStandart([in] CSLTMVideoStandart std) – sets video format (STD_PAL – PAL, STD_NTSC – NTSC) for all inputs and outputs of the FDTM board.

HRESULT get_VideoStandart([out] CSLTMVideoStandart* std) – returns a current video format (STD_PAL or STD_NTSC).

HRESULT set_VideoSize([in] CSLTMVideoSize VideoSize) – sets horizontal resolution (SLTM_FullFull or SLTM_HalfFull can be set).

HRESULT get_VideoSize([out] CSLTMVideoSize* VideoSize, [out] long *dx, [out] long *dy) – returns current horizontal resolution (SLTM_FullFull or SLTM_HalfFull) and frame size dx and dy. For example, for PAL and SLTM_HalfFull resolution dx=352 and dy=576 will be returned.

HRESULT get_InputValue([in] SLTMInputValuesType Index, [out] long *pVal) – returns a value of an input parameter (see remark 1).

HRESULT set_InputValue([in] SLTMInputValuesType Index, [in] long Val) – sets a value of an input parameter (see remark 1).

HRESULT GetInputValueRange([in] SLTMInputValuesType Index, [out] LONG* min, [out] LONG* max, [out] LONG* def) – returns a permissible range and default value of a specified input parameter (see remark 1).

HRESULT get_OutputValue([in] SLTMOutputValuesType Index, [out] long *pVal) – returns a value of an output parameter (see remark 1).

HRESULT set_OutputValue([in] SLTMOutputValuesType Index, [in] long Val) – sets a value of an output parameter (see remark 1).

HRESULT GetOutputValueRange([in] SLTMOutputValuesType Index, [out] LONG* min, [out] LONG* max, [out] LONG* def) – returns permissible range and default value of specifed output parameter (see remark 1).

HRESULT GetInputsList( [out, size_is(cb/sizeof(SLTM_IO_Desc))] SLTM_IO_Desc *List, [in] long cb, [out] long* prcb, [in] long DecoderIndex) – returns (fills-in) the array of structures, describing all possible (including not allowed for using) inputs (see remark 3).

HRESULT GetOutputsList([out, size_is(cb/sizeof(SLTM_IO_Desc))] SLTM_IO_Desc *List, [in] long cb, [out] long* prcb) – returns (fills-in) the array of structures, describing all possible (including not allowed for using) outputs (see remark 4).

HRESULT set_GenlockType([in] sltmGenlock_t Type) – the same as set_OutputValue (tmc_output_slave, Type).

HRESULT get_GenlockType([out] sltmGenlock_t* Type) – the same as get_OutputValue (tmc_output_slave, Type).

HRESULT set_DigitalIO1([in] sltm_digital_io1_t Val) – the same as set_InputValue (tmc_input_dio, Val).

HRESULT get_DigitalIO1([out] sltm_digital_io1_t* Val) – the same as get_InputValue (tmc_input_dio, Val).

HRESULT set_DigitalIO2([in] sltm_digital_io2_t Val) – the same as set_OutputValue (tmc_output_dio, Val).

HRESULT get_DigitalIO2([out] sltm_digital_io2_t* Val) – the same as get_OutputValue (tmc_output_dio, Val).

Remarks:

1. List of input parameters (settings of the FDTM board inputs):

tmc_input_standart	PAL or NTSC video format (STD_PAL or STD_NTSC correspondingly)

tmc_input_1_type	identifier of input for the first decoder (line A) (see remark 3)

tmc_input_2_type	identifier of input for the second decoder (line B) (see remark 3)

tmc_input_vtrc	not used

tmc_input_vpref	not used

tmc_input_vcross	not used

tmc_input_vhue	not used

tmc_input_vsat	not used

tmc_input_vbright	not used

tmc_input_vcontr	not used

tmc_input_vcory	not used

tmc_input_vaper	not used

tmc_input_vband	not used

tmc_input_vnoise	not used

tmc_input_gain	not used

tmc_input_u_gain	not used

tmc_input_v_gain	not used

tmc_input_gain_lock	not used

tmc_input_vhue_1	hue of the first decoder (line A)

tmc_input_vhue_2	hue of the second decoder (line B)

tmc_input_vsat_1	saturation of the first decoder (line A)

tmc_input_vsat_2	saturation of the second decoder (line B)

tmc_input_vbright_1	brightness of the first decoder (line A)

tmc_input_vbright_2	brightness of the second decoder (line B)

tmc_input_vcontr_1	contrast of the first decoder (line A)

tmc_input_vcontr_2	contrast of the second decoder (line B)

tmc_input_u_gain_1	gain of U component of the first decoder (line A) (for the FD200 board only)

tmc_input_u_gain_2	gain of U component of the second decoder (line B) (for the FD200 board only)

tmc_input_v_gain_1	gain of V component of the first decoder (line A) (for the FD200 board only)

tmc_input_v_gain_2	gain of V component of the second decoder (line B) (for the FD200 board only)

tmc_chroma_delay_1	chroma signal delay (UV signals) relative to Y signal of the first decoder (line A) (for the FD200 board only)

tmc_chroma_delay_2	chroma signal delay (UV signals) relative to Y signal of the second decoder (line B) (for the FD200 board only)

tmc_hstart_1	offset of clipping window for input image while working with half horizontal resolution of the first decoder (line A) (in range from 0 to 720-352*2-1=15)

tmc_hstart_2	offset of clipping window for input image while working with half horizontal resolution of the second decoder (line B) (in the range from 0 to 720-352*2-1=15)

tmc_input_dio	source of video data for «input» digital bus (40-pin) of FDTM board: tmc_digital_io1_none – data come from an «outside» source – for example, from the DVM board; tmc_digital_io1_output_1 – data come from the first decoder (line A); tmc_digital_io1_output_2 – data come from the second decoder (line B).

2. List of the output parameters (settings of the FDTM board output):

tmc_output_slave	genlock source: tmc_genlock_master – no genlock, tmc_genlock_a – genlock from the first decoder (line A), tmc_genlock_b – genlock from the second decoder (line B).

tmc_output_adapter_type	identifier of the output.

tmc_genlock_clocks_delay	horizontal offset of the output signal (delay) relative to genlock source.

tmc_genlock_lines_delay 	vertical offset of the output signal (delay) relative to genlock source.

tmc_output_dio	data source for «output» digital bus (26-pin) of the FDTM board: tmc_digital_io2_none – bus is not active (in the 3rd state), data from DSP TriMedia come to the output coder only; tmc_digital_io2_output – data from DSP TriMedia come to the output coder and outside, to output bus; tmc_digital_io2_input – data come from «outside», from an external source – for example, from the DVM board, then go to output coder, and data from DSP TriMedia do not go out (in the DSP TriMedia’s application there are no output interrupts).

3. The description of the FDTM board input is represented by the array of the SLTM_IO_Desc structures (maximal number of the FDTM board inputs – 32 inputs):

typedef struct SLTM_IO_Desc

{

long	dsc[8];

char	s[64];

} SLTM_IO_Desc;

The ‘s’ string contains a text description of the input. The first value dsc[0] – the identifier of an input (a value of the CdecoderInputs type that is used in the set_InputValue and get_InputValue funсtions with the first parameter set to tmc_input_1_type or tmc_input_2_type). The second value dsc[1] – the type of an input (a value of the sltmVideoAnalogAdapter_t type). The other fields are currently not used. For example, for the 11th composite input set: dsc[0] = SLTM_CVBS11, dsc[1] = SLTMvaaCVBS, s = «CVBS12».

4. The description of the FDTM board outputs is represented by the array of the SLTM_IO_Desc structures (maximal number of the FDTM board outputs – 16 outputs). The ‘s’ string contains a text description of the output. The first value dsc[0] – the identifier of an output (a value of the sltmVideoAnalogAdapter_t type that is used in the set_OutputValue and get_OutputValue functions with the first parameter set to tmc_output_adapter_type). The second value dsc[1] – the type of an output (a value of the sltmVideoAnalogAdapter_t type). The other fields are currently not used. For example, the default output is described by the following values: dsc[0] = SLTMvaaColorBarYUV, dsc[1] = SLTMvaaColorBarYUV, s = «ColorBar YUV + CVBS». For output RGB+Sync: dsc[0] = SLTMvaaRGBSync, dsc[1] = SLTMvaaRGB, s = « RGB + Sync ».

## ISLTMArtist

Object: CLSID_SLTMSrv

HRESULT set_ArtistDataSource([in] CSLTM_ArtistDataSource Val) – sets a type of input data (see remarks 1 and 2).

HRESULT get_ArtistDataSource([out] CSLTM_ArtistDataSource* pVal) – returns a type of input data.

Remarks:

1. The FDTM board contains two video decoders. Any of them can be active and used as a source of video data for an artist: its video data are color-corrected (or are not) and are used for construction of alpha-channel. At first, during video input to FIFO the first 16 pixels come from an active decoder, and then from an inactive decoder (see the description of the RGBAlphaFormat format in remarks for the description of the ISLTMGraber interface).

2. The types of output data of the FDTM board:

SLTM_ASA	YUV data from the first decoder only. The first decoder is active.

SLTM_ASB	YUV data from the second decoder only. The second decoder is active.

SLTM_ASAAlfa	YUV+Alpha data from the first decoder only. The first decoder is active.

SLTM_ASBAlfa	YUV+Alpha data from the second decoder only. The second decoder is active.

SLTM_ASAAlfaB	YUV+Alpha data from the first decoder and YUV data from the second decoder. The first decoder is active. In the SLTM_MOFulllMix mixer mode data from the second decoder will be used as scene data.

SLTM_ASBAlfaA	YUV+Alpha from the second decoder and YUV data from the first decoder. The second decoder is active. In the SLTM_MOFulllMix mixer mode data from the first decoder will be used as scene data.

SLTM_ASAB	YUV data from the first and second decoder. The first decoder is active.

SLTM_ASBA	YUV data from the first and second decoder. The second decoder is active.

## ISLTMAutoKey

Object: CLSID_SLTMAutoKey

HRESULT MakeAutoKey([in] IUnknown *pIServer, [in] long nLineIndex) – performs automatic tuning of the color-keying parameters for a specified decoder (the nLineIndex parameter) and board (the pIServer parameter must point to the SLTMSrv object of a desired board).

## ISLTMKeyer

Object: CLSID_SLTMSrv

HRESULT LoadKeyTable([in, size_is(cb)] BYTE* KeyTable, long cb, long bankIndex) – loads a keying table into the first (bankIndex=0) or second (bankIndex=1) bank. The keying table is an array DWORD KeyTable[64*64*64], where address in the table is constructed by higher 6 bits of each of Y, U, and V components, and data are presented by 4-byte values of resulting color – Y, U, V, and Alpha. If Y, U, and V values in the table are less than 16 (that is prohibited for the color values and is never present in real data), these values are ignored and Y, U, and V components are used directly from the input.

HRESULT set_VFilter([in] CSLTMAlfaVFilter Val) – sets the vertical filtration mode of alpha-channel (currently is not used and will be performed by the software in the DSP TriMedia or in PC).

HRESULT get_VFilter([out] CSLTMAlfaVFilter* pVal) – returns a current mode of the vertical filtration of alpha-channel.

HRESULT set_HFilter([in] CSLTMAlfaHFilter Val) – sets a mode of the horizontal filtration of alpha-channel (is performed by the hardware before data input to DSP TriMedia).

HRESULT get_HFilter([out] CSLTMAlfaHFilter* pVal) – returns a current mode of the horizontal filtration of alpha-channel.

HRESULT set_ColorCorrection([in] long On) – turns the color-correction mode on/off (the use of keying table for changing of Y, U, V components).

HRESULT get_ColorCorrection([out] long* On) – returns 1, if color-correction is on, and 0 otherwise.

HRESULT set_AlfaEdge([in] long Min, [in] long Max) – sets a scaling range of alpha-channel values:

alpha_out = alpha_in <= Min ? 0 : (alpha_in >= Max ? 255 : (alpha_in-Min)*255/(Max-Min));

(is performed in DSP TriMedia and currently not performed during data input from PC).

HRESULT get_AlfaEdge([out] long* Min, [out] long* Max) – returns a current scaling range of alpha-channel.

HRESULT set_KeyTableBank([in] long Val) – switches an active keying table to the first (0) or second (1) bank.

HRESULT get_KeyTableBank([out] long* pVal) – returns index of an active keying table.

## – ISLTMDVMboard

Object: CLSID_SLTMSrv

HRESULT get_DVMState([out] DVM_State *status) – returns a current status of the DVM board, connected to «input» bus of the FDTM board via 40-pin connector:

DVM_Absent	board is not connected.

DVM_ReadOnly	board is connected, but is used in the «Standalone» mode.

DVM_Available	board is connected and available for control.

HRESULT ReadDVMInfo() – reads a status of the DVM board to internal variables.

HRESULT WriteDVMInfo() – writes a status to the DVM board from internal variables.

HRESULT get_DVMValue([in] DVMParameterType Index, [out] long *pVal) – returns a value of a specified internal variable (see remark 1).

HRESULT set_DVMValue([in] DVMParameterType Index, [in] long Val) – sets a value of a specified internal variable (see remark 1).

HRESULT get_DVMModeDescription([in] DVMMode Mode, [out] DVMSource *analog, [out] DVMSource *sdi, [out] DVMSource *digital40, [out] DVMSource *digital26) – returns a description of data sources for the analog output, SDI output (sdi), large 40-pin (digital40), and narrow 26-pin (digital26) digital buses in a specified mode of the DVM board (see remark 1).

Important: Before you start reading the variable values using get_DVMValue, you must get them from the DVM board using the ReadDVMInfo function. After setting desired values of variables using the WriteDVMInfo function, you should call the WriteDVMInfo function to write variable values to the DVM board.

Remarks:

- 1. The types of internal variables describing a status of the DVM board:

DVM_Format	format (a value of the DVMFormat type)

DVM_Mode	mode (a value of the DVMMode type), its description can be obtained using the get_DVMModeDescription funtion

DVM_YUVOutputType	type of the analog output of the DVM board (a value of the DVMOutput type)

DVM_Flicker	value of vertical anti-flicker filter (from 0 to 16)

DVM_Sharpness	value of horizontal sharpness filter (from 0 to 31)

DVM_Hoffset	horizontal offset of image (from 0 to 511 pixels)

DVM_Voffset	vertical offset of image (from 0 to 511 lines)

DVM_DVIInputHSyncInvert	on/off inverting of HSync signal of the DVI output

DVM_DVIInputVSyncInvert	on/off inverting of VSync signal of the DVI output

DVM_DVIInputBlankInvert	on/off inverting of Blank signal of the DVI output

- 2. The types of data sources of the DVM board output:

DVM_SourceDVI	from the DVI input

DVM_SourceSDI	from the DVI output

DVM_SourceFD200	from the FDTM board via narrow bus (26-pin)

DVM_SourceNONE	no data (output is inactive)

## – ISLTMTesting

Object: CLSID_SLTMSrv

HRESULT get_Cnts([out] all_cnts_type* pCnts, [in] LONG cb, [out] LONG* n) – returns an array with special statistical data.

HRESULT get_DiagsCnts([out] all_cnts_type* pCnts, [in] LONG cb, [out] LONG* n) – returns an array with special statistical data.

HRESULT get_I2C([in] LONG i2c_address, [in] LONG i2c_sub_address, [out] LONG *pValue) – performs reading of a register on the IIC bus.

HRESULT set_I2C([in] LONG i2c_address, [in] LONG i2c_sub_address, [in] LONG value) – performs writing to a register on the IIC bus.

HRESULT get_I2Cfocus([in] LONG i2c_address, [in] LONG i2c_sub_address, [out] LONG *pValue) – performs reading of a register on the IIC bus using a non-standard protocol (compatible with protocol of Focus chip of the DVM board).

HRESULT set_I2Cfocus([in] LONG i2c_address, [in] LONG i2c_sub_address, [in] LONG value) – performs writing to a register on the IIC bus using a non-standard protocol (compatible with protocol of Focus chip of the DVM board).

## x ISLTMGraber

Object: CLSID_SLTMSrv

HRESULT get_GrabFormat([in] CSLTMDataSource DataSource, [out, size_is(cb)] BYTE* Buffer, [in] long cb) – returns data format to a specified buffer (the sltmVideoFormat_t structure, see remark 1) for input (DTP_MixerInputA or DTP_MixerInputB – effect is the same) or output (DTP_MixerOutput) video data.

HRESULT CreateFifo([in] long Size, [in] long MinSize) – creates FIFO for video capture (cyclic buffer in the PC memory) with a specified Size in bytes. FIFO size is always aligned to 4096 bytes. If there is no possibility to create FIFO of a specified Size (insufficient physical memory), the object decreases Size to 0.5 Mb and tries to allocate memory again. It tries to do it until Size is equal to MinSize. If there is no possibility to allocate FIFO with size of MinSize, the function will fail.

HRESULT CreateFifo2([in] long Size, [in] long MinSize, [in] long TailSize) – the same as the previous function, but FIFO is allocated with «tail» size TailSize. It can be useful for copying video data from the beginning of FIFO to this tail to get continuous frame data (for its post-processing, for example).

HRESULT ReleaseFifo() – releases the previously allocated FIFO (if any).

HRESULT GetFifoInfo([out] BYTE **Address, [out] long *FifoSize) – returns pointer to FIFO data start (Address) and real size (FifoSize) allocated for video capture.

HRESULT StartGrabStreaming([in] CSLTMDataSource DataSource) – starts video capturing from a specified video data source.

HRESULT StopGrabStreaming() – stops video capturing.

HRESULT GetStreamInfo([out] long *Offset, [out] long *Busy) – returns information about video data already captured to FIFO – data offset from the beginning of FIFO and full captured data size. The data can «overflow» FIFO and get over the end of FIFO – it is possible that Offset + Busy > FifoSize. When data reach FIFO end, they will be continued at the beginning of FIFO.

HRESULT CopyFromStream([out, size_is(Size)] BYTE *Buffer, [in] long Offset, [in] long Size) – copies data to a user buffer from FIFO with check of data «overflow». Note that current position of free space start in FIFO does NOT change (see next function).

HRESULT ShiftStreamPos([in] long Size) – shifts start position in FIFO that is processed and can be used for capturing of new video data. The start position of free space in FIFO is shifted by this function only.

HRESULT SynchroSwitch([in] CSLTM_ArtistDataSource DataSource, [in] long KeyTableBank) – switches active input channel to first (DTP_MixerInputA) or second (DTP_MixerInputB) decoder and active keying table to first (0) or second (1) bank.

HRESULT CaptureFields([in] long count) – captures a specified number of fields from a current input. The function will return only when the operation is complete. This function (unlike StartGrabStreaming) does not require registration that enables video stream capturing.

Remarks:

1. The sltmVideoFormat_t structure that is returned during a video capturing process:

CSLTMRGBAlfaFormat RGBAlphaFormat;

CSLTMVideoStandart videoStandard;

long imageWidth;

long imageHeight;

long imageStride;

long imageUVStride;

long activeVideoStartX;

long activeVideoStartY;

long activeVideoEndX;

long activeVideoEndY;

unsigned long flags;

sltmTimeStamp_t time;

The RGBAlphaFormat parameter defines data format in frame. For output data it can be SLTM_FYUV422 – 3 data fragments: all Y, all U, and all V values. For input data it can be SLTM_F16YUV, SLTM_F16YAUV, SLTM_F16YUVYUV or SLTM_F16YAUVYUV – one data fragment, each 16 pixels correspond to 16 Y of an active channel (from an active decoder), 16 Y of an inactive channel (from an inactive decoder), 16 A (alpha/mask/non-transparency) from an active channel, 8 U of an active channel, 8 V of an active channel, 8 U of an inactive channel, 8 V of an inactive channel. If all this data are denoted as Y1,Y2,A1,U1,V1,U2,V2, then SLTM_F16YUV will be transferred as Y1+U1+V1; SLTM_F16YAUV – as Y1+A1+U1+V1; SLTM_F16YUVYUV – as Y1+Y2+U1+V1+U2+V2 and SLTM_F16YAUVYUV – as Y1+Y2+A1+U1+V1+U2+V2.

The videoStandard parameter defines a video standard (PAL/NTSC) (really is not used).

The imageWidth and imageHeight parameters define a field size in pixels.

The imageStride parameter defines a line size in bytes for the first data fragment.

The imageUVStride parameter defines a line size in bytes for the second and third data fragments.

The activeVideoStartX, activeVideoStartY, activeVideoEndX, and activeVideoEndY parameters are not used.

The flags parameter contains a set of bit flags. The only flag is really useful – sltmField2, it indicates that the current field is odd.

The time parameter defines a field count number (64-bit number).

## x ISLTMMixer

Object: CLSID_SLTMSrv

HRESULT set_MixerOutput([in] CSLTMMixerOutput Val) – sets a video mixer mode (see remark 1).

HRESULT get_MixerOutput([out] CSLTMMixerOutput* Val) – returns a current video mixer mode.

HRESULT set_BackgroundAlfaUsage([in] long On) – not used.

HRESULT get_BackgroundAlfaUsage([out] long* On) – not used.

HRESULT set_ArtistAlfaUsage([in] long On) – not used.

HRESULT get_ArtistAlfaUsage([out] long* On) – not used.

Remarks:

1. The video mixer modes of the FDTM board:

SLTM_MOArtist	shows an artist only (video data from an active decoder).

SLTM_MOArtistAndAlfa	shows an artist and alpha-channel (for output mode with separate alpha channel, for example, for SLTMvaaYUV_Alpha).

SLTM_MOAlfa	shows alpha channel only (as black-and-white image).

SLTM_MOBackground	shows a scene only (video data from PC).

SLTM_MOFulllMix	mixes an artist (video data from active decoder) with a scene (video data from PC) using alpha-channel of the artist. This mode is useful only for input data type SLTM_ASAAlfa/SLTM_ASBAlfa (video scene is taken from PC) or SLTM_ASAAlfaB/SLTM_ASBAlfaA (video scene is taken from an inactive decoder).

## x ISLTMBackground

Object: CLSID_SLTMSrv

HRESULT CreatePlaybackFifo([in] long Size, [in] long MinSize) – creates FIFO for video playback (cyclic buffer in the PC memory) with a specified Size in bytes. FIFO size is always aligned to 4096 bytes. If there is no possibility to create FIFO of a specified Size (insufficient physical memory), the object decreases Size to 0.5 Mb and tries to allocate memory again. It tries to do it until Size is equal to MinSize. If there is no possibility to allocate FIFO with size of MinSize, the function will fail.

HRESULT CreatePlaybackFifo2([in] long Size, [in] long MinSize, [in] long TailSize) – the same as previous function, but FIFO is allocated with «tail» size of TailSize. It can be useful to copy full video fields to FIFO before playback and then copy part of last field that exceeds FIFO end to the beginning of FIFO.

HRESULT ReleasePlaybackFifo() – releases previously allocated playback FIFO.

HRESULT GetPlaybackFifoInfo([out] BYTE **Address, [out] long *FifoSize) – returns pointer to the beginning of allocated FIFO and real size of memory allocated for playback on the FDTM board.

HRESULT StartYUVPlayback() – starts playback of uncompressed video data (using one of the formats used for video capture via the ISLTMGraber interface) from FIFO to the FDTM board.

HRESULT StartMPEGPlayback() – starts playback of MPEG-compressed video data from FIFO to the FD200 board (it is required to use the KARM.OUT application in DSP TriMedia, works with half horizontal resolution only). Not supported by the FDTM board.

HRESULT StopPlayback() – stops video playback.

HRESULT GetPlaybackInfo([out] long *Offset, [out] long *Free) – returns information about free space in playback FIFO – offset from the beginning of FIFO and fill size of free space. Data fragment can get over the end of FIFO, so it is possible that Offset + Free > FifoSize.

HRESULT CopyToPlayback([in, size_is(Size)] BYTE *Buffer, [in] long Size) – copies video data from a buffer in PC memory to playback FIFO with the check of FIFO «overflow», and automatically shifts start position of free space in FIFO (to size of copied data).

HRESULT ShiftPlaybackPos([in] long Size) – shifts the start position of free space in FIFO to a specified value (it is expected that user has manually copied video data to FIFO before).

HRESULT SynchroSwitchAtFrame([in] CSLTM_ArtistDataSource DataSource, [in] long KeyTableBank, [in] long FifoFrame) – switches an active video decoder and corresponding active keying table while mixing a specified field (with a number starting from the start of playback). Currently is not supported by the FDTM board.

HRESULT PauseFifo([in] long abs_position) – not used.

HRESULT UnpauseFifo() – not used.

HRESULT GetFifoState([out] tmslFIFOReadState_t *state, [out] long *abs_position) – not used.

## ISLTMSound

Object: CLSID_SLTMSrv

HRESULT SndCheckVersion() – returns S_OK, if the versions of the application in PC and FDTM board correspond to each other.

HRESULT get_SndStatus([out] ULONG *pStatus) – returns a status of audio subsystem of the FDTM board:

eSndStatusFail = -1			// Sound device is not present

eSndStatusOk = 0			// Sound device is present and stopped

eSndStatusOpened = 1	// Sound device is present and opened

eSndStatusStarted = 2	// Sound device is present and audio is started

HRESULT SndStart([in] ULONG SampleRate, [out] ULONG* pErr) – starts audio subsystem of the FDTM board with a specified sound sample rate.

HRESULT SndStop() – stops audio subsystem of the FDTM board.

HRESULT get_SndRatio([out] ULONG *pRatio) – returns a current sound sample rate.

HRESULT put_SndDelayMs([in] int InChannel, [in] ULONG DelayMs) – sets an audio delay value (in milliseconds) for a specified mono-input (from 0 to 5).

HRESULT get_SndDelayMs([in] int InChannel, [out, retval] ULONG *pDelayMs) – returns an audio delay value (in milliseconds) for a specified mono-input (from 0 to 5).

HRESULT put_SndMonoMixVolume([in] int InCh, [in] int OutCh, [in] LONG vol) – sets a level (see remark 1) of audio mixing from a specified mono-input (from 0 to 5) to a specified mono-output (from 0 to 5).

HRESULT get_SndMonoMixVolume([in] int InCh, [in] int OutCh, [out, retval] LONG *pvol) – returns a level (see remark 1) of audio mixing from specified a mono-input (from 0 to 5) to specified a mono-output (from 0 to 5).

HRESULT put_SndMonoOutVolume([in] int OutChannel, [in] LONG vol) – sets a hardware sound volume (see remark 1) for a specified mono-output.

HRESULT get_SndMonoOutVolume([in] int OutChannel, [out, retval] LONG *pvol) – sets a hardware sound volume (see remark 1) for a specified mono-output.

HRESULT put_SndMonoInVolume([in] int InChannel, [in] LONG vol) – sets a hardware sound volume (see remark 1) for a specified mono-input.

HRESULT get_SndMonoInVolume([in] int InChannel, [out, retval] LONG *pvol) – returns a hardware sound volume (see remark 1) for a specified mono-input.

HRESULT put_SndMixMute([in] int InCh, [in] int OutCh, [in] ULONG mute) – disables (1) or enables (0) mixing of audio data from a specified mono input to a specified mono output.

HRESULT get_SndMixMute([in] int InCh, [in] int OutCh, [out, retval] ULONG *pmute) – returns the mute state of a specified mono input to a specified mono output.

HRESULT put_SndOutMute([in] int OutChannel, [in] ULONG mute) – disables (1) or enables (0) output of audio data to a specified mono-output.

HRESULT get_SndOutMute([in] int OutChannel, [out, retval] ULONG *pmute) – returns the mute state of a specified mono-output.

HRESULT put_SndInMute([in] int InChannel, [in] ULONG mute) – disables (1) or enables (0) input of audio data from a specified mono-input.

HRESULT get_SndInMute([in] int InChannel, [out, retval] ULONG *pmute) – returns the mute state of a specified mono-input.

HRESULT put_SndInBoost([in] int InChannel, [in] ULONG boost) – turns on (1) or off (0) additional 20 dB boost for a specified mono-input. Can be used for inputs 2/3 and 4/5 only, and can be set for both inputs together at the same time only: 2 and 3 or 4 and 5 correspondingly.

HRESULT get_SndInBoost([in] int InChannel, [out, retval] ULONG *pboost) – returns 1, if additional 20 dB boost is turned on for a specified mono-input.

The following functions are used in case when you want to work with the stereo sound sources only. But even in this case we recommend to use the mono-versions of these functions.

HRESULT put_SndMixVolume([in] int InCh, [in] int OutCh, [in] ULONG vol, [in, defaultvalue(0x80000000)] ULONG crossVol) – sets a level (see remark 1) of mixing of audio data from a specified stereo-input (see remark 2) to a specified stereo-output (see remark 2). The vol and crossVol levels set two volume levels: LOWORD(vol) – the mixing level of the left input channel to the left output channel, HIWORD(vol) – the level of mixing of the right input channel to the right output channel, LOWORD(crossVol) – the level of mixing of the left input channel to the right output channel, HIWORD(crossVol) – the level of mixing of the right input channel to the left output channel. Note that by default the value of crossVol is 0x80000000 – special value. The sound mixer of the FDTM board does not change current settings of cross-mixing (from left channel to right and vice-versa) in this case, so if you want to disable cross-mixing, set volume to its minimal value – MAKELONG(MinVolume,MinVolume).

HRESULT get_SndMixVolume([in] int InCh, [in] int OutCh, [out] ULONG *pvol, [out, defaultvalue(0)] ULONG *pCrossVol) – returns a level (see remark 1) of mixing of audio data from a specified stereo-input (see remark 2) to a specified stereo-output (see remark 2).

HRESULT put_SndOutVolume([in] int OutChannel, [in] ULONG vol) – sets a hardware sound volume (see remark 1) of a specified stereo-output (see remark 2).

HRESULT get_SndOutVolume([in] int OutChannel, [out, retval] ULONG *pvol) – returns a hardware sound volume (see remark 1) of a specified stereo-output (see remark 2).

HRESULT put_SndInVolume([in] int InChannel, [in] ULONG vol) – sets a hardware sound volume (see remark 1) of a specified stereo-input (see remark 2).

HRESULT get_SndInVolume([in] int InChannel, [out, retval] ULONG *pvol) – returns a hardware sound volume (see remark 1) of a specified stereo-input (see remark 2).

Remarks:

1. All volume values are specified in values of 0.1 dB. The minimal volume value (absolute silence) is –960 (-96.0 dB, MinVolume constant). The maximal volume value is 120 (+12.0 dB, MaxVolume constant, equal to 4-times gain) for all volume values, except of the hardware volume levels of the output channels – their maximal volume is 0 dB. The «Normal» volume (no change in sound) is 0 (0.0 dB, NormalVolume constant).

2. For convenient work with stereo-data the inputs and outputs are grouped to stereo pairs:

stereo-channel 0 = mono-channel 0 + mono-channel 1,

stereo-channel 1 = mono-channel 2 + mono-channel 3,

stereo-channel 2 = mono-channel 4 + mono-channel 5.

# Enclosure

## 1. Typical working modes of the FDTM board’s video mixer

| Mode | The set_MixerOutput parameter of the ISLTMMixer interface | The set_ArtistDataSource parameter of the ISLTMArtist interface | Type of output |
| --- | --- | --- | --- |
| Mixing (and color-keying) of a live artist with video-decorations from PC | SLTM_MOFulllMix | SLTM_ASAAlfa or SLTM_ASBAlfa | 1 |
| Mixing (and color-keying) of a live artist with video-decorations from PC (from an inactive decoder) | SLTM_MOFulllMix | SLTM_ASAAlfaB or SLTM_ASBAlfaA | 1 |
| Artist input (color-corrected image from an active decoder with alpha-channel) | SLTM_MOArtistAndAlfa | SLTM_ASAAlfa or SLTM_ASBAlfa | 2 |
| Video input from an active decoder without color-correction and without alpha-channel | SLTM_MOArtist | SLTM_ASA or SLTM_ASB | 1 |
| Titles superimposition (full-screen decorations with transparency) | SLTM_MOFulllMix | SLTM_ASA or SLTM_ASB | 1 |
| Playback of video-decorations from PC without alpha-channel (ordinary movie) | SLTM_MOBackground | any | 1 |
| Playback of video-decorations from PC with alpha-channel | SLTM_MOBackground | any | 2 |

Types of output:

1)	Ordinary outputs without alpha-channel (single output), such as «Y/C+CVBS», «YUV+CVBS», «YUV+Sync», «RGB+Sync». The value of the tmc_output_adapter_type parameter of the set_OutputValue function of the ISLTMIOSettings interface can be one of the following: SLTMvaaCVBS, SLTMvaaSvideo, SLTMvaaYC, SLTMvaaYUV, SLTMvaaRGB, SLTMvaaColorBarYC, SLTMvaaColorBarYUV, SLTMvaaColorBarRGB, SLTMvaaYUVSync or SLTMvaaRGBSync.

2)	Outputs with both image and alpha-channel (double output), such as «YUV+Key», «RGB+Key». The value of the tmc_output_adapter_type parameter of the set_OutputValue function of the ISLTMIOSettings interface can be one of the following: SLTMvaaSvideo_Alpha, SLTMvaaYUV_Alpha, SLTMvaaRGB_Alpha or SLTMvaaRgB_Alpha.

## 2. Video Stream Input

To input the continuous sequence of uncompressed images, it’s better to use the SLTMSimpleGrabber object with the ISimpleGrabber interface. This example uses the C++ language with ATL. The error handling is not represented to save the space and to facilitate comprehension of the SDK work principles, but it is strongly recommended to implement error handling while calling the SDK’s functions – as it greatly simplifies the debugging procedure of your code (checked in practice). Moreover, if you send us some part of your code with the error code, you assist us in understanding of your problem.

Note, that video data input is implemented by fields, i.e. 50 frames per second, image size 720x288 for PAL video standard or 60 frames per second, image size 720x240 for NTSC standard.

Consider the process step by step:

- COM Initialization

As SDK is written using the COM-objects, therefore, you should initialize the COM-library in every thread of execution, for example by calling the CoInitializeEx function (NULL, COINIT_MULTITHREADED). At the end of the work it’s required to call the CoUninitialize() function. Note that by the moment of calling this function you should close ALL COM-objects, especially the static objects, which are destroyed after the termination of the main() procedure and, correspondingly, after the call of the CoUninitialize() function.

- Object Initialization

The Grabber object is required for operation:

CComPtr<ISimpleGrabber> pIGrabber;

Usually, the Grabber object is a class variable, which can be used in various procedures. The object creation:

hr = pIGrabber.CoCreateInstance(CLSID_SLTMSimpleGrabber,NULL,CLSCTX_INPROC_SERVER);

Then select the board index you are going to work:

long count;
hr = pIGrabber->GetBoardCount(&count);
long index = {0..count-1}; // select a desired index
hr = pIGrabber->AcquireBoard(index);

You can specify the current parameters of the board (image size and frame rate):

long dx,dy; double frame_rate;
hr = pIGrabber->GetBoardInfo(&dx,&dy,&frame_rate);

If the image size and frame rate do not meet your requirements, you can change them (refer to the following chapter). Then set the internal parameters of your application to work with the frame size data (specify the data array for storing the frame of the calculated size, etc.)

- Stream Start

You may start stream from the input or output of the TriMedia mixer. It’s defined by the first parameter of the following function:

hr = pIGrabber->StartStream(SG_Artist, 15, FALSE);

The SG_Artist parameter means the data input from the input (or SG_MixOut – for data input from the mixer output). If it’s required to input data from the first or second video input, you may specify the first parameter SG_Artist_1 (or SG_Artist_2)

- Data Receipt

When the stream is started, trace the fullness of FIFO.

long captured, free, first, last;
hr = pIGrabber->GetStreamInfo(&captured, &free, &first, &last);

Here the main field for you is the captured field: how many fields are in FIFO. As soon as it becomes more, than zero, you can get the data. For example, you want to receive the general data for displaying using GDI resources (24 bits per pixel):

// dy – field height 
#define BITMAP_WIDTH(dx, bits) (((dx*bits+31)/32)*4)
int stride = BITMAP_WIDTH(dx,24);
BYTE buffer = new BYTE[dy*stride];
hr = GetFieldImage(0, 0, 0, dx, dy, SG_RGB, buffer, dy*stride, -stride);

The stride parameter has a negative value, therefore, the data in your buffer will be vertically inverted. If you want to work with frames, and not with fields, then you should wait for several fields (at least two fields) to appear in FIFO and get the information about their parity. Every field in FIFO has its own index in order starting with zero. When the first field is deleted from FIFO, the indices of the subsequent fields will be diminished by one. You can get the information about every field in FIFO by specifying its index:

long FieldNumber, dx, dy; SGSource Src; SGFieldPosition FieldPos;
hr = GetFieldInfo(index, &FieldNumber, &Src, &FieldPos, &dx, &dy);

Moreover, you can also get the information about the real image size, which might be changed, if you or someone else has modified the board settings.

Having received the information about the fields, you may obtain the whole frame into your buffer by specifying what fields should be regarded as even and odd ones:

// dy – frame hieght
BYTE buffer = new BYTE[dy*dx*2];
hr = GetFrameImage(upper, lower, 0, 0, dx, dy, SG_UYVY, buffer, dy*dx*2, dx*2);

where upper – field’s index with the upper part of the frame (the SGFieldPosition parameter is equal to SG_Top), and lower – field’s index with the lower part of the frame (the SGFieldPosition parameter is equal to SG_Bottom), the YUV422 format – means that the output buffer will contain the data in the following order: UYVYUYVYUYVY….

As soon as the required fields are processed, you must skip them, i.e. inform the object that they can be used to input new fields:

hr = pIGrabber->SkipFields(1);

or

hr = pIGrabber->SkipFields(2);

You can also skip all frames:

hr = pIGrabber->SkipFields(SG_SkipAll);

or you can skip all frames except the last one:

hr = pIGrabber->SkipFields(SG_SkipAllExceptLastField);

If you do not call this function, then FIFO will be overfilled, and all subsequent frames won’t be recorded into FIFO.

After the call of this function the frames’ indices in FIFO will be diminished by a specified number; i.e. if you save the parameters of each field in FIFO of your local data array, then this information should be cyclically moved to a specified number of fields.

- Stop Stream

hr = pIGrabber->StopStream();

- Object Release

pIGrabber = NULL;
pIServer = NULL;

For proper operation of the object the FD300 board should be registered with the enabled input from the specified source. If the input is not enabled, all functions will return S_OK, but none of the fields will ever appear in FIFO. In this case the following function can be used:

hr = pIGrabber->CaptureFields(SG_Artist, 5, FALSE);

it inputs the specified number of fields in FIFO (but no more, than 10 fields). This function is very suitable for preview, when the receipt of one or several frames is sufficient for setting the keying parameters, selecting video effects and line, etc.

Recommendations

If the FD300 board does not receive the output frame interruptions, then the data input stop in the Host PC is possible. The lack of frame interruption can be caused by the no-signal on the bus, which synchronizes. For example, if you set the first line as a genlock source, and a very poor synchrosignal comes to it. Or if you set the Master genlock mode, the output bus is assigned to video data input (e.g. from the DVM board), and actually the video signal does not come to the bus (the DVM board does not work or there is no data source for the DVM board). In this case there won’t be proper synchrosignal on the output of the FD300 board, and respectively, there won’t be guaranteed (secured) frame interruptions. The input process of next video frame to TriMedia is not activated (enabled) because of the frame interruption lack, and therefore, the input process of video data to the Host PC stops.

## 3. Standard Selection (image size and frame rate)

To change the image size and video standard PAL/NTSC, initialize the COM-library, create the COM-object – pIGrabber and initialize it, as described at the beginning of the previous chapter.

Then request the ISLTMIOSettings interface from the created object and call the function of the video standard setting, for example PAL:

CComQIPtr<ISLTMIOSettings> pIIO(pIGrabber);
hr = pIIO->set_VideoStandart(STD_PAL); // or STD_NTSC for NTSC

Then you can change the image width, select either full or half resolution:

hr = pIIO->set_VideoSize(SLTM_FullFull); // or SLTM_HalfFull for half resolution

## 4. Showing Property Pages

To show the property pages of the FDTM board, you can use the standard OleCreatePropertyFrame function. First, create an instance of special property link object that will be given to property page objects.

CComPtr<ISLTMPropertyLink> link;

HRESULT hr = link.CoCreateInstance(CLSID_SLTMPropertyLink);

Then initialize it by giving real data to it:

link->put_Server(pIServer);

link->put_Line(0);

link->put_Restriction(PL_BoardIndex|PL_LineIndex|PL_CheckSecurity|PL_FieldHHalfRes);

The first parameter (pIServer) – reference to SLTMSrv (or SLTMSimpleGrabber) object.

The second parameter – line number (decoder index), for which you want to show parameters.

The third parameter – mask of restrictions and special settings, that define type of behavior of property pages.

Then create an array with class identifiers of property page objects that you want to show to user:

CLSID page[4];

page[0] = CLSID_LinePropertyPage;

page[1] = CLSID_FormatPropertyPage;

page[2] = CLSID_OutputPropertyPage;

page[3] = CLSID_DVMPropertyPage;

Finally launch a property frame with selected property pages:

LPUNKNOWN objects = link;

hr = OleCreatePropertyFrame(m_hWnd,20,20, L"FDTM Board", 1,&objects, 4,page, 0,0,NULL);

After the dialog is closed, you will be able to know if some parameters are changed via property link object:

long changed = 0;

link->get_Changed(&changed);

## 5. Saving/restoring state of FDTM board

The current parameters of the FDTM board can be saved to the system registry. In order to do this, initialize COM library (if you didn’t do it before), create an instance of the SLTMSrv object and initialize it (select desired board) as it is done in the first two sections of Enclosure “2. Video Stream Input”. Then, you should create an instance of the TMSLPropertyBag object:

CComPtr<ITMSLPropertyBagSettings> pIBagSettings;
hr = pIBagSettings.CoCreateInstance(CLSID_TMSLPropertyBag);

Then you need to set path in the registry where you want to store board settings:

CComBSTR nameRoot("SOFTWARE\\MyCompany\\MyApplication\\FDTM_Board_1");
hr = pIBagSettings->SetRegistryKey((DWORD)HKEY_CURRENT_USER,nameRoot);
CComQIPtr<IPropertyBag> pIBag(pIBagSettings);

Use pIBag object to save all parameters of the FDTM board via the SLTMSrv object:

CComQIPtr<ITMSLContextSaveRestore> pISaveRestore(pIServer);
hr = pISaveRestore->Save(pPBag);

Or you can save only some part of parameters corresponding to one or more PropertyPages (see Enclosure “4. Showing Property Pages”):

CComQIPtr<ITMSLContextSaveRestore> pISaveRestore(pIServer);
hr = pISaveRestore->SavePropertyPageInfo(PL_LinePage, pPBag);	// LinePropertyPage
hr = pISaveRestore->SavePropertyPageInfo(PL_FormatPage, pPBag);	// FormatPropertyPage
hr = pISaveRestore->SavePropertyPageInfo(..., pPBag);

Where in the first parameter of the SavePropertyPageInfo function one of the following values is used:

PL_LinePage,	// LinePropertyPage

PL_FormatPage,	// FormatPropertyPage

PL_OutputPage,	// OutputPropertyPage

PL_DVMPage,	// DVMPropertyPage

PL_SoundPage	// SoundPropertyPage

- If you need to restore previously saved parameters, use the Load and LoadPropertyPageInfo methods instead of Save and SavePropertyPageInfo of the same interface.

- There is an example of two procedures: the first one for saving and the second one for restoring settings of all FDTM boards installed in the system. The settings are saved in the system registry (local machine – for all users, change HKEY_LOCAL_MACHINE to other root key if you need it).

#define SLTM_SUBKEY_PATH "SOFTWARE\\Darim Vision\\SLTM\\BoardSettings"

// Save all Boards’ settings

HRESULT SaveSettings()

{

HRESULT hr;

// Create local SLTM server for board enumeration

CComPtr<ISLTMSrv> pLocalSLTMSrv;

hr = pLocalSLTMSrv.CoCreateInstance(CLSID_SLTMSrv);

if (FAILED(hr)) return hr;

// Get context save/restore interface

CComPtr<ITMSLContextSaveRestore> pContextSave;

hr = pLocalSLTMSrv.QueryInterface(&pContextSave);

if (FAILED(hr)) return hr;

// Create property bag settings object

CComPtr<ITMSLPropertyBagSettings> pPropBagSettings;

hr = pPropBagSettings.CoCreateInstance(CLSID_TMSLPropertyBag);

if (FAILED(hr)) return hr;

// Get property bag interface

CComPtr<IPropertyBag> pPropBag;

hr = pPropBagSettings.QueryInterface(&pPropBag);

if (FAILED(hr)) return hr;

// Enumerate all boards

long nBoards;

hr = pLocalSLTMSrv->get_BoardsCount(&nBoards);

for (long i=0; i<nBoards; i++) {

// Try to select board #i

hr = pLocalSLTMSrv->set_SelectedBoardIndex(i);

if (FAILED(hr)) return hr;

// Create subkey name for board #i

char subkey[1024] = SLTM_SUBKEY_PATH;

char index[128]; wsprintf(index,"\\Board%d",i);

strcat(subkey,index);

// Set registry key to property bag settings

CComBSTR KeyStr(subkey);

hr = pPropBagSettings->SetRegistryKey((DWORD)HKEY_LOCAL_MACHINE,KeyStr);

if (FAILED(hr)) return hr;

// Save settings for current board ("Format" and "Output" pages only)

hr = pContextSave->SavePropertyPageInfo(PL_FormatPage,pPropBag);

if (FAILED(hr)) return hr;

hr = pContextSave->SavePropertyPageInfo(PL_OutputPage,pPropBag);

if (FAILED(hr)) return hr;

}

return hr;

}

// Restore all Boards’ settings

HRESULT CVideoRenderer::RestoreSettings()

{

HRESULT hr;

// Create local SLTM server for board enumeration

CComPtr<ISLTMSrv> pLocalSLTMSrv;

hr = pLocalSLTMSrv.CoCreateInstance(CLSID_SLTMSrv);

if (FAILED(hr)) return hr;

// Get context save/restore interface

CComPtr<ITMSLContextSaveRestore> pContextSave;

hr = pLocalSLTMSrv.QueryInterface(&pContextSave);

if (FAILED(hr)) return hr;

// Create property bag settings object

CComPtr<ITMSLPropertyBagSettings> pPropBagSettings;

hr = pPropBagSettings.CoCreateInstance(CLSID_TMSLPropertyBag);

if (FAILED(hr)) return hr;

// Get property bag interface

CComPtr<IPropertyBag> pPropBag;

hr = pPropBagSettings.QueryInterface(&pPropBag);

if (FAILED(hr)) return hr;

// Enumerate all boards

Sleep(100); // Pause 100 ms – sometimes restoring doesn't work without it

long nBoards;

hr = pLocalSLTMSrv->get_BoardsCount(&nBoards);

for (long i=0; i<nBoards; i++) {

// Try to select board #i

hr = pLocalSLTMSrv->set_SelectedBoardIndex(i);

if (FAILED(hr)) return hr;

// Create subkey name for board #i

char subkey[1024] = SLTM_SUBKEY_PATH;

char index[128]; wsprintf(index,"\\Board%d",i);

strcat(subkey,index);

// Set registry key to property bag settings

CComBSTR KeyStr(subkey);

hr = pPropBagSettings->SetRegistryKey((DWORD)HKEY_LOCAL_MACHINE,KeyStr);

if (FAILED(hr)) return hr;

// Load settings for current board ("Format" and "Output" pages only)

hr = pContextSave->LoadPropertyPageInfo(PL_FormatPage,pPropBag);

if (FAILED(hr)) return hr;

hr = pContextSave->LoadPropertyPageInfo(PL_OutputPage,pPropBag);

if (FAILED(hr)) return hr;

}

return hr;

}

You may also add saving/restoring of a currently selected board number (refer to the description of the ISLTMSrv interface), and change a name of registry sub-key in SLTM_SUBKEY_PATH define. Note that this code saves/restores state of the variables that are present in the “Format” and “Output” property pages only for each board. Use the Load and Save methods if you want to save/restore all parameters.

## 6. Format of video data in FIFO

The video frames are stored in FIFO by fields. The following structure precedes each field:

typedef struct sltmPackHeader {

long n;

struct { long offset, size; } items[8];

} slPackHeader_t;

This structure describes the number of elements in a current field, their size and offset from the beginning of the structure. The first element is the slPackHeader_t structure itself. The second element always contains a field format:

typedef struct {

CSLTMRGBAlfaFormat	RGBAlphaFormat;

CSLTMVideoStandart	videoStandard;

long	imageWidth;

long	imageHeight;

long	imageStride;

long	imageUVStride;

long	activeVideoStartX;

long	activeVideoStartY;

long	activeVideoEndX;

long	activeVideoEndY;

unsigned long	flags;

sltmTimeStamp_t	time;

} sltmVideoFormat_t;

The RGBAlphaFormat item describes type of video data in a current field (one of the values SLTM_F16YUV, SLTM_F16YAUV, SLTM_F16YUVYUV or SLTM_F16YAUVYUV for video data captured from input of the FDTM board and SLTM_FYUV422 for data captured from the internal mixer). The videoStandard item describes video format PAL/NTSC. The imageWidth and imageHeight items describe a field size in pixels. The imageStride item describes size of one video line in bytes (offset from start of one line to start of next line). If the field is divided by three data fragments (video data from the output), the imageStride item describes size of line with lightness components (first data fragment), and imageUVStride item – size of line with U and V components (second and third data fragments). The activeVideoStartX, activeVideoStartY, activeVideoEndX and activeVideoEndY items are not used. The flags item contains some field attributes, for example, the sltmField2 flag signifies that a current field is odd (corresponds to SG_Bottom). This item also contains the CSLTM_PacketFlags bit flags that can be used to know what video decoder has been active while generating a current field. And finally, the time item contains count number of field from the start of application in the FDTM board.

The structures are followed by field video data itself. For input FIFO (video data from «Artist»), all data are packed into one fragment, and the number of fragments in the field is 3. Input video data (from FPGA Spartan) are always aligned to 16 pixels. The maximal complexive video stream contains the following sequence for each 16 pixels: 16 Y1, 16 Y2, 16 A, 8 U1, 8V1, 8 U2, 8 V2 (80 bytes max for each 16 pixels), where A – alpha channel, Y1, U1 and V1 – video data from the first (active) decoder, Y2, U2 и V2 – video data from the second (inactive) decoder. The order of data in sequence is always the same, but depending on the type of data (see the set_ArtistDataSource method of the ISLTMArtist interface) some part of data can be omitted. For example, while capturing video data from an active decoder with alpha channel (the parameter of the set_ArtistDataSource method is set to SLTM_ASAAlfa or SLTM_ASBAlfa), the following sequence is generated: 16Y1 + 16A + 8U1 + 8V1. Full stream contains 48 bytes for each 16 pixels, that is 3 bytes per pixel. While capturing video data from an active decoder without color-keying (and without alpha-channel) (the parameter of the set_ArtistDataSource method is set to SLTM_ASA or SLTM_ASB) the other sequence is generated: 16Y1 + 8U1 + 8V1, that is 32 bytes for each 16 pixels, and 2 bytes per pixel.

While capturing video data from the output of video mixer, the field is separated by three fragments (after two structures slPackHeader_t and sltmVideoFormat_t) – all lightness components (the third fragment), all U-components (the fourth fragment) and all V-components (the fifth fragment). So, each field in FIFO contains 5 data fragments. The line size in bytes for the third fragment (Y-component) is set in imageStride, and for 4-th and 5-th (U- and V-components) is set in imageUVStride.

The fields are stored one-by-one in FIFO, forming circular buffer. Since the size of FIFO and size of one field are not divisible, data in one field can be separated and get over the end of FIFO: the first part of field can be stored at the end of FIFO, and the second part of field – at the start of FIFO.

## 7. Four-input videomixer and video playback

The SLTMSrv object provides access to 4-input videomixer of the FDTM board via ISLTMMixer4 and ISLTMMixer4Inputs interfaces. Video data from the inputs are considered as (in the order of input indexes from 0 to 3) the following video streams:

- Input 0 – video stream A (artist, or foreground)

- Input 1 – video stream B (background)

- Input 2 – alpha channel α (alpha mask for artist)

- Input 3 – Title (image with alpha-channel)

The mixer mixes video streams in the following order: puts video stream A (artist from Input 0) over video stream B (background from Input 2) via alpha-channel (from Input 1) and puts titles (from Input 3) over the result of mix.

Video-mixer inputs can be logically attached to «scene» objects that are sources of video data (there can be up to 4 scenes), or to current inbut of the board («Live» video). Scenes get video data from FIFO buffers in the memory of PC (there can be up to 16 FIFO buffers), and data can be being loaded to FIFO from hard disk for example.

So, each input of video mixer can be attached to one scene or to current board input (and one scene can be attached to several mixer inputs simultaneously), and in order to get video data each active scene must be attached to FIFO buffer in the PC memory.

ISLTMPlayback interface of SLTMSrv object can be used for creation of basic objects. It allows creation of Scene, FIFO and Timeline object; last one can be attached to mixer input instead of FIFO object. Timeline object allows creation of sequence of video clips with the use of simple functions, so it is highly recommended to use it in the majority of cases. Clips in the Timeline can be of the following formats:

- File in AVI/WAV formats (video and/or audio), entire file or fragment of specified duration

- Still image in one of JPEG, Targa, Bitmap, PNG formats

- Empty clip (repeats last image) of specified duration

- Solid-color background of specified color and duration

- Special clip that turns on transport of “live” video from input to output

- Group, or sequence of clips, or fragment of such sequence with specified duration (SuperClip)

- Video-fragment previously recorded with PostPlay server

Clips can be added only to the end of Timeline, but it can be done even during playback (only if current playback time didn’t reach start time of a clip being added). Important: any clip parameters may not be changed after clip is added to Timeline! For example, you must not load other file to a clip.

For each type of file clip there is a special object that is used to load file of corresponding format. In order to add loaded file to Timeline, a reference to such object must be transferred to Timeline.

| Clip type | Object and interface for file loading | Methods of ISLTMAddMedia interface for clip adding |
| --- | --- | --- |
| File in AVI/WAV format or fragment of specified duration | CLSID_SLTMPlayMovie, ISLTMPlayMovie | AddMovie, AddMovieFragment |
| Still image from graphic file (in one of JPEG, Targa, BMP, PNG formats) of specified duration | CLSID_SLTMPlayImage, ISLTMPlayImage | AddImage |
| Empty clip of specified duration | – | AddEmpty |
| Solid-color backgound of specified color and duration | – | AddFlat |
| Live video from current board input (with specified duration) | – | AddLive |
| Group (sequence) of clips or fragment of such sequence with specified duration | CLSID_SLTMSuperClip, ISLTMSuperClip | AddSuperClip, AddSuperClipFragment |

An example of source code that creates timeline and plays video clip with the use of timeline is presented in the following enclosure.

## 8. Example: playback of AVI file using Timeline

In order to play an AVI file with the use of Timeline object, the following sequence of actions should be performed:

- Create SLTMSrv object and select desired FDTM board with it (via ISLTMSrv interface).

- Get ISLTMPlayback interface from SLTMSrv object and create “scene” object (ISLTMPlaybackScene interface) and Timeline object (ISLTMTimeline interface) with specified video-FIFO size (at least some begabytes is required). In order to play video with sound, audio-FIFO size is also specified (at least 1 Mb is recommended).

- Create ISLTMPlayMovie object and load desired videoclip (AVI-file) in it with OpenFile method.

- Get ISLTMAddMedia interface from Timeline object and add clip to Timeline with AddMovie method.

- Set Timeline to “Prepare to playback” state with put_State method.

- Attach Timeline to Scene object with AttachFifo method.

- Get ISLTMMixer4Inputs interface from SLTMSrv object.

- Attach Scene object to input A (artist, or foreground) of 4-channel videomixer.

- Set Timeline to “Playback” state with put_State method.

Is is strongly recommended to check result of each operation (return value of HRESULT  type) with FAILED macro, and dicontinue working if any method returned an error.

Below there is an example of source code of procedure that perform all actions mentioned above (full path to AVI-file is passed in the FileName parameter; bPlayWithAudio parameter must be set to TRUE if audio should be played too):

HRESULT SLTMPlayAVIFile(LPCTSR FileName, BOOL bPlayWithAudio)

{

// Create SLTMSrv object

CComPtr<ISLTMSrv> pSrv;

HRESULT hr = pSrv.CoCreateInstance(CLSID_SLTMSrv);

if (FAILED(hr)) return hr;

// Get number of FDTM boards

long nb;

hr = pSrv->get_BoardsCount(&nb);

if (FAILED(hr)) return hr;

// If there is no boards present, return error

if (nb < 1) return E_FAIL;

// Select board with index 0

hr = pSrv->set_SelectedBoardIndex(0);

if (FAILED(hr)) return hr;

// Get ISLTMPlayback interface

CComPtr<ISLTMPlayback> pPlayback;

hr = pSrv.QueryInterface(&pPlayback);

if (FAILED(hr)) return hr;

// Create Scene object

CComPtr<ISLTMPlaybackScene> pScene;

hr = pPlayback->CreateScene(&pScene,0);

if (FAILED(hr)) return hr;

// Create Timeline object

CComPtr<ISLTMTimeline> pTimeline;

if (bPlayWithAudio) // play video with audio on default audio device

{

// video-fifo size 8 Mb, min size 1 Mb, audio-fifo size 1 Mb

hr = pPlayback->CreateAVTimeline(8192*1024,1024*1024,1024*1024,-2,&pTimeline,0);

if (FAILED(hr)) return hr;

}

else // play video without audio

{

// video-fifo size 8 Mb, min size 1 Mb

hr = pPlayback->CreateTimeline(8192*1024,1024*1024,&pTimeline,0);

if (FAILED(hr)) return hr;

}

// Create Playmovie object

CComPtr<ISLTMPlayMovie> pPlayMovie;

hr = pPlayMovie.CoCreateInstance(CLSID_SLTMPlayMovie);

if (FAILED(hr)) return hr;

// Get filename in BSTR format

CComBSTR bFileName(FileName);

// Open the file

hr = pPlayMovie->OpenFile(bFileName.m_str);

if (FAILED(hr)) return hr;

// Get ISLTMAddMedia interface

CComPtr<ISLTMAddMedia> pMedia;

hr = pTimeline.QueryInterface(&pMedia);

if (FAILED(hr)) return hr;

// Add clip to Timeline

hr = pMedia->AddMovie(pPlayMovie,SLTMPlayAnyMovie);

if (FAILED(hr)) return hr;

// Prepare Timeline to playback

hr = pTimeline->put_State(SLTMPlaybackPrepared);

if (FAILED(hr)) return hr;

// Attach Timeline to Scene object

hr = pScene->AttachFifo(pTimeline);

if (FAILED(hr)) return hr;

// Get ISLTMMixer4Inputs interface

CComPtr<ISLTMMixer4Inputs> pMixer;

hr = pSrv.QueryInterface(&pMixer);

if (FAILED(hr)) return hr;

// Attach Scene object to input A of video-mixer (foreground)

hr = pMixer->AttachScene2Input(pScene,SLTMMixerInputArtist);

if (FAILED(hr)) return hr;

// Launch playback of Timeline

hr = pTimeline->put_State(SLTMPlaybackPlaying);

if (FAILED(hr)) return hr;

// Wait until user stops playback

MessageBox("Press OK to finish playback");

return hr;

}