- Description of matrix configuration format

At start the FDOnAir application reads settings from the registry folder "HKEY_LOCAL_MACHINE\SOFTWARE\SoftLab-NSK\FDSwitcher_v1.0".

In this folder COM port parameters are specified as the following keys:

"PortName" – a COM port name string.

"PortRate" – number specifying port rate (bit per second)

"PortBits" – number specifying digits per word (in bits)

"PortStopBitsEnum" – number specifying stop bits type:

- 0 – 1 stop bit

1 – 1.5 stop bits

2 – 2 stop bits

"PortParityEnum" – number specifying parity check:

- 0 – none

1 – Odd Parity

2 – Even Parity

3 – Mark Parity

4 – Space Parity

Then a number of various switches is specified:

"SwitchCount" – number of subfolders with individual switches settings.

Each switch is saved in an individual folder named "SwitchN", where "N" is substituted with the switch sequence number beginning from "1".

Each subfolder should include the following keys:

"SwitchName" – switch user name string.

"SwitchID" – switch unique identifier (any number), which must always remain the same. It is used for saving and restore settings. It is impossible to use the same identifiers for different switches.

"DataCount" – number of control sequences that should be sent to a COM port on the switch selection (activation).

"SendDelayN" – number specifying how many fields (half-frames) before the start of displaying a real video the control sequence should be sent. The letter "N" in the key name is replaced by sequence number. Usually advance (or delay) value is limited by 6 (3 frames) because of real switching time limited by 3-4 frames on interactive input switching.

"SendDataN" – the control sequence to be sent to a COM port. The letter "N" in the key name is replaced by sequence number.

"AnswerDataN" – data sequence that should be sent by the matrix in response to the control sequence. The letter "N" in the key name is replaced by sequence number. The application attempts to read the specified sequence from COM port but no longer than during one frame. It is not required to specify this sequence.

All switches read from the registry are displayed in the "Switch" folder of the line settings dialog in the FDOnAir application. The first name specified in the list is always "None", that means that it is not required to send anything to COM port on the line activation. If something wrong in the switch format in the registry then the corresponding switch will be skipped in the list of available switches in the FDOnAir live settings.

In the Kramer matrix manual the following COM port settings are specified:

Rate = 9600, Bits = 8, ParityCheck = None, StopBits = One stop-bit.

These correspond with the following registry settings:

"PortRate" = 9600

"PortBits" = 8

"PortStopBitsEnum" = 0

"PortParityEnum" = 0

- An example file with two switches for Kramer matrix

- Windows Registry Editor Version 5.00

[HKEY_LOCAL_MACHINE\SOFTWARE\SoftLab-NSK\FDSwitcher_v1.0]

"PortName"="COM1"

"PortRate"=dword:00002580

"PortBits"=dword:00000008

"PortStopBitsEnum"=dword:00000000

"PortParityEnum"=dword:00000000

"SwitchCount"=dword:00000002

[HKEY_LOCAL_MACHINE\SOFTWARE\SoftLab-NSK\FDSwitcher_v1.0\Switch1]

"SwitchName"="My test 1->1"

"SwitchID"=dword:00000001

"DataCount"=dword:00000001

"SendDelay1"=dword:00000003

"SendData1"=hex:01,81,81,81

"AnswerData1"=hex:81,81,81,81

[HKEY_LOCAL_MACHINE\SOFTWARE\SoftLab-NSK\FDSwitcher_v1.0\Switch2]

"SwitchName"="My test 2->1"

"SwitchID"=dword:00000002

"DataCount"=dword:00000001

"SendDelay1"=dword:00000003

"SendData1"=hex:01,82,81,81

"AnswerData1"=hex:81,82,81,81

Установки в FDOnAir для управления матрицей

Для управления матрицей из программы FDOnAir необходимо вначале настроить параметры переключения матрицы соответствующими записями в реестре. Затем необходимо назначить для каждого входа соответствующее переключение матрицы. Обратите внимание, что одному входу в программе FDOnAir соответствует только одно переключение! Для выбора переключения необходимо зайти в настройки входов, разрешить нужный вход и нажать на кнопку справа от переключателя входа. По этой кнопке вызывается диалог настройки параметров входа, который включает три закладки: Input, Switch и Tally:

Настройки параметров входа: выбор входной линии

Закладка Input позволяет выбрать конкретную входную линию и настроить ее параметры (яркость, контраст и т.п.).

Настройки параметров входа: выбор переключения матрицы

Закладка Switch позволяет выбрать подготовленное ранее переключение матрицы из списка "Switcher line".

Настройки параметров входа: выбор GPI-out сигнала для индикации показа входа

Закладка Tally позволяет выбрать GPI-out сигнал, который будет активироваться при отображении настраиваемого входа.